EFW design response to Deep Dielectric Discharge Events as per 7417-9018 EMECP.

LVPS

J701

| Pin | Name | Special Implementation for DDD |
| --- | --- | --- |
| 1,9 | INST_P28V | Deemed not necessary. Pulse absorbed by L3 & C5 |
| 2,10 | INST_P28V_RTN | Deemed not necessary. Pulse absorbed by C9 |
| 3,8,11 | CHASSIS | Directly connected to chassis |
| 4,12 | SPB_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 5,13 | SPB_P28V_RTN | 1nF 100V Ceramic capacitor added between net and Chassis |
| 6,14 | AXB_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 7,15 | AXB_P28V_RTN | 1nF 100V Ceramic capacitor added between net and Chassis |

J705

| Pin | Name | Special Implementation |
| --- | --- | --- |
| 1 | AXB_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 2 | DOOR_P6 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 3 | AXB_P28V_RTN | Implemented on J701 |
| 4 | MOTOR6_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 5 | AXB_P28V_RTN | 1nF 100V Ceramic capacitor added between net and Chassis |
| 6 | TEMP5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 7 | BEB_AGND | Directly connected to chassis |
| 8 | DOOR_P5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 9 | AXB_P28V_RTN | Implemented on J701 |
| 10 | MOTOR5_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 11 | AXB_P28V_RTN | Implemented on J701 |
| 12 | N/C | Pin not installed |
| 13 | MOTOR4_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 14 | DOOR1_SMA_RTN | Implemented on J701 |
| 15 | MOTOR3_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 16 | DOOR1_SMA_RTN | Implemented on J701 |
| 17 | N/C | Pin not installed |
| 18 | MOTOR2_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 19 | DOOR1_SMA_RTN | Implemented on J701 |
| 20 | MOTOR1_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 21 | DOOR1_SMA_RTN | Implemented on J701 |
| 22 | AXB_P28V_RTN | Implemented on J701 |
| 23 | DOOR_S5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 24 | AXB_P28V_RTN | Implemented on J701 |
| 25 | STACER_P6 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 26 | AXB_P28V_RTN | Implemented on J701 |
| 27 | AXB_P28V | 1nF 100V Ceramic capacitor added between net and Chassis |
| 28 | AXB_P28V_RTN | Implemented on J701 |
| 29 | DOOR_S5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 30 | AXB_P28V_RTN | Implemented on J701 |
| 31 | STACER_P5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 32 | AXB_P28V_RTN | Implemented on J701 |
| 33 | N/C | Pin not installed |
| 34 | DOOR4_SMA | 1nF 100V Ceramic capacitor added between net and Chassis |
| 35 | DOOR1_SMA_RTN | Implemented on J701 |
| 36 | DOOR3_SMA | 1nF 100V Ceramic capacitor added between net and Chassis |
| 37 | DOOR1_SMA_RTN | Implemented on J701 |
| 38 | N/C | Pin not installed |
| 39 | DOOR2_SMA | 1nF 100V Ceramic capacitor added between net and Chassis |
| 40 | DOOR1_SMA_RTN | Implemented on J701 |
| 41 | DOOR1_SMA | 1nF 100V Ceramic capacitor added between net and Chassis |
| 42 | DOOR1_SMA_RTN | Implemented on J701 |
| 43 | TEMP6 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 44 | BEB_AGND | Directly connected to chassis |
| 45 | SENSE6 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 46 | BEB_AGND | Directly connected to chassis |
| 47 | STACER_S5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 48 | AXB_P28V_RTN | Implemented on J701 |
| 49 | SENSE5 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 50 | BEB_AGND | Directly connected to chassis |
| 51 | STACER_S5 | 1nF 100V Ceramic capacitor added between net and Chassis |
| 52 | AXB_P28V_RTN | Implemented on J701 |
| 53 | N/C | Pin not installed |
| 54 | SENSE4 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 55 | BEB_AGND | Directly connected to chassis |
| 56 | SENSE3 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 57 | BEB_AGND | Directly connected to chassis |
| 58 | N/C | Pin not installed |
| 59 | SENSE2 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 60 | BEB_AGND | Directly connected to chassis |
| 61 | SENSE1 | 10nF 100V Ceramic capacitor added between net and BEB_AGND |
| 62 | BEB_AGND | Directly connected to chassis |

DCB

J702

| Pin | Name | Special Implementation for DDD |
| --- | --- | --- |
| 1,3,6,7 | To UT54LVDS032LV | APL Tested. Meets DDD. |
| 4,9 | To UT54LVDS031LV | APL Tested. Meets DDD. |
| 2,5,8 | IDPU_DGND | Directly connected to chassis |

DFB

J703

| Pin | Name | Special Implementation for DDD |
| --- | --- | --- |
| 1 | SCM_X | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 2 | SCM_X_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 3 | (spare) | Pin not installed |
| 4 | SCM_Y | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 5 | SCM_Y_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 6 | (spare) | Pin not installed |
| 7 | SCM_Z | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 8 | SCM_Z_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 9 | (spare) | Pin not installed |
| 10 | (spare) | Pin not installed |
| 11 | (spare) | Pin not installed |
| 12 | (spare) | Pin not installed |
| 13 | (spare) | Pin not installed |
| 14 | (spare) | Pin not installed |
| 15 | (spare) | Pin not installed |
| 16 | (spare) | Pin not installed |
| 17 | (spare) | Pin not installed |
| 18 | (spare) | Pin not installed |
| 19 | MAG_X | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 20 | MAG_X_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 21 | (spare) | Pin not installed |
| 22 | MAG_Y | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 23 | MAG_Y_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 24 | (spare) | Pin not installed |
| 25 | MAG_Z | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| 26 | MAG_Z_Shield | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |

J715-J720

| Pin | Name | Special Implementation for DDD |
| --- | --- | --- |
| J715 | V1 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_003 |
| J716 | V2 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_004 |
| J717 | V3 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_005 |
| J718 | V4 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_006 |
| J719 | V5 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_007 |
| J720 | V6 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_008 |

J706, J707, J708

| Pin | Name | Special Implementation for DDD |
| --- | --- | --- |
| 1 | FV2_GND | 1nF protects output from BEB |
| 2 | FVF2_M15VA | 0.1uF protects output from BEB |
| 3 | GUARD2 | 3300pF protects output from BEB |
| 4 | BIAS2 | 3300pF protects output from BEB |
| 5 | EMFISIS_1_2 | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_002 |
| 6 | USHER1 | 1nF protects output from BEB |
| 7 | ACTEST1 | Input to AC14, tested for DDD. Test Report RBSP_EFW_TR_012 |
| 8 | FVF1_P15VA | 0.1uF protects output from BEB |
| 9 | VSPHERE1 | 10nF protects input to BEB |
| 10 | VSPHERE2 | 10nF protects input to BEB |
| 11 | FVF2_P15VA | 0.1uF protects output from BEB |
| 12 | ACTEST2 | Input to AC14, tested for DDD. Test Report RBSP_EFW_TR_012 |
| 13 | USHER2 | 1nF protects output from BEB |
| 14 | N/C | Pin not installed |
| 15 | BIAS1 | 3300pF protects output from BEB |
| 16 | GUARD1 | 3300pF protects output from BEB |
| 17 | FVF1_M15VA | 0.1uF protects output from BEB |
| 18 | FV1_GND | 1nF protects output from BEB |
| 19 | AGND | Directly connected to chassis |
| 20 | AGND | Directly connected to chassis |
| 21 | AGND | Directly connected to chassis |
| 22 | AGND | Directly connected to chassis |
| 23 | EMF_1-2_RET | Input to AD648, tested for DDD. Test Report RBSP_EFW_TR_002 |
| 24 | AGND | Directly connected to chassis |
| 25 | AGND | Directly connected to chassis |
| 26 | AGND | Directly connected to chassis |

Preamp

All preamp inputs have been tested for DDD, see RBSP_EFW_CCR_012. Preamp is also cleared for surface charging issues as per telecom with APL / Carl Hermnan 9/23/09 from the testing that was done for the waiver.

SPB

The spin plane boom only contains 1 semiconductor device, a 5W 1N5811 diode. These are protected by the inductor / filters on the motor power and return lines.

A technical note detailing all external insulators was also written up (TN_029) to detail ESC issues.

AXB

The axial boom only contains 1 semiconductor device, a 5W 1N5811 diode. These are protected by the inductor / filters on the motor power and return lines.