Dielectric Withstanding Voltage Tests

Per MIL-STD-202G Method 301 / MIL-STD-1344A Method 3001.1 (connectors).

Test apparatus:  	Associated Research 3670 Hypot III tester Serial Number 9331523, calibrated 3/28/2008.

Test Operator:  	David Curtis

Test Date:	2008/4/18

Test Article:	CAY1398 slip rings (unit used for ETU tests). Bruce Dalen soldered on wire extensions.  Voltage applied between wires or between pins and case with remote end open.

Settings:	Test M2: 2 sec ramp-up (DC test), 5 sec dwell, 2μA overcurrent trip (to capture arcing), Arc detect set to 9, followed by a 2s 900V >10Gohm IR test.  Test at Ambient.

| Test | VDC | Application | Results |
| --- | --- | --- | --- |
| 1 | 900 | Red wire +, other 11 wires Gnd, case open | Pass |
| 2 | 900 | All wires except red +, Fixed case Gnd. Red wire, rotating case open | Fail, DWV during ramp up at ~10V, I=112uA. With a DVM isolate problem to Brown wire, which has 80 ohms to fixed case. |
| 3 | 900 | Red wire +, fixed case -, all others open | Pass |
| 4 | 900 | All but brown wire +, fixed case -, brown wire open | Pass |
| 5 | 900 | All but brown wire +, brown wire -, case open | Pass |

Comments:  	This unit has been through some testing prior to this.  Will repeat on a fresh unit and also try disassembling this unit to determine what cases failure during test #2.  Note that the case is divided into three parts electrically: fixed part, rotating part, and shaft; these mostly have high impedance, with intermittent moderate (100kohm) impedance when rotated.