Dielectric Withstanding Voltage Tests

Per MIL-STD-202G Method 301 / MIL-STD-1344A Method 3001.1 (connectors).

Test apparatus:  	Associated Research 3670 Hypot III tester Serial Number 9331523, calibrated 3/28/2008.

Test Operator:  	David Curtis

Test Date:	2008/4/22

Test Article:	CAY1398 slip rings, marked S/N 028, “Flight”.  Voltage applied between wires or between wires and case with remote end open.

Settings:	Test M2: 2 sec ramp-up (DC test), 60 sec dwell, 2μA overcurrent trip (to capture arcing), Arc detect set to 9 (max sensitivity), followed by a 2s 900V >10Gohm IR test.  Test at Ambient.

Pre-test:	With DVM verify continuity across slip-rings, isolation wires to 3 parts of case.  3 parts of case intermittently isolated.

| Test | VDC | Application | Results |
| --- | --- | --- | --- |
| 1 | 900 | All wires +, Fixed case - | Pass |
| 2 | 900 | All wires +, Rotating case - | Pass |
| 3 | 900 | All wires +, Shaft - | Pass |
| 4 | 900 | Black, Red, Yellow, Blue, Gray, Black/Brown +, Brown, Orange, Green, Purple, White, Black/Red -, case open | Pass |
| 5 | 1000 | All wires +, Fixed case - | Pass |
| 6 | 1000 | All wires +, Rotating case - | Pass |
| 7 | 1000 | All wires +, Shaft - | Pass |
| 8 | 1000 | Black, Red, Yellow, Blue, Gray, Black/Brown +, Brown, Orange, Green, Purple, White, Black/Red -, case open | Pass |
| 9 | 1000 | Black +, remaining wires -, case floats | Pass |
| 10 | 1000 | Brown +, remaining wires -, case floats | Pass |
| 11 | 1000 | Red +, remaining wires -, case floats | Pass |
| 12 | 1000 | Orange +, remaining wires -, case floats | Pass |
| 13 | 1000 | Yellow +, remaining wires -, case floats | Pass |
| 14 | 1000 | Green +, remaining wires -, case floats | Pass |
| 15 | 1000 | Blue +, remaining wires -, case floats | Pass |
| 16 | 1000 | Purple +, remaining wires -, case floats | Pass |
| 17 | 1000 | Grey +, remaining wires -, case floats | Pass |
| 18 | 1000 | White +, remaining wires -, case floats | Pass |
| 19 | 1000 | Brown/Black +, remaining wires -, case floats | Pass |
| 20 | 1000 | Red/Black +, remaining wires -, case floats | Pass |

Comments:  	Note that with 4x derating on DWV, these parts are good to at least 250VDC, which exceeds the requirements for the RBSP EFW.