Dielectric Withstanding Voltage Tests

Per MIL-STD-202G Method 301 / MIL-STD-1344A Method 3001.1 (connectors).

Test apparatus:  	Associated Research 3670 Hypot III tester Serial Number 9331523, calibrated 3/28/2008.

Test Operator:  	David Curtis

Test Date:	2008/4/23

Test Article:	Omnetics MCS12S/MCP12S connectors (mated), with pigtails added by Bruce Dalen.  Voltage applied between wires at one end with remote end isolated.

Settings:	Test M2: 2 sec ramp-up (DC test), 60 sec dwell, 2μA overcurrent trip (to capture arcing), Arc detect set to 9 (max sensitivity), followed by a 2s 900V >10Gohm IR test.  Test at Ambient.

| Test | VDC | Application | Results |
| --- | --- | --- | --- |
| 1 | 500 | Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass |
| 2 | 900 | Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass |
| 3 | 1000 | Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass |
| 4 | 1000 | MCP12S Unmated (source side in application). Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass |
| 5 | 1000 | Black +, remaining wires - | Pass |
| 6 | 1000 | Brown +, remaining wires - | Pass |
| 7 | 1000 | Red +, remaining wires - | Pass |
| 8 | 1000 | Orange +, remaining wires - | Pass |
| 9 | 1000 | Yellow +, remaining wires - | Pass |
| 10 | 1000 | Green +, remaining wires - | Pass |
| 11 | 1000 | Blue +, remaining wires - | Pass |
| 12 | 1000 | Purple +, remaining wires - | Pass |
| 13 | 1000 | Grey +, remaining wires - | Pass |
| 14 | 1000 | White +, remaining wires - | Pass |
| 15 | 1000 | Brown/Black +, remaining wires - | Pass |
| 16 | 1000 | Red/Black +, remaining wires - | Pass |
| 17 | 700VAC (1000 Vp-p) | Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass (17uA) |
| 18 | 1200V | Black, Brown, Red, Orange, Yellow, Green +; Blue, Purple, Grey, White, Brown/Black, Red/Black - | Pass |

Comments:  	Note that with 4x derating on DWV, these parts are good to at least 250VDC, which exceeds the requirements for the RBSP EFW.