# TN-004.  Sensor Noise Levels

## General.

The EFW team has been reviewing the EMFISIS (IOWA) noise requirements with respect to theoretical performance of the EFW baseline sensors. The following introduces the concerns of the EFW team.:

- IOWA’s sensitivity requirement of 3x10-17 (V/m)2/Hz at 3 kHz (LVL2EMF15) is a very difficult requirement to meet since the EFW sensor is dominated by shot noise in the wave domain.

- EFW-proposed sensitivities with SPB’s of 100 m tip-to-tip / 3 m fine wire is a challenge on its own. Careful attention to power supply noise etc will be necessary to achieve low noise levels.

- Historical data from FAST shows FAST was dominated by power supply noise. Remember that, on FAST, we decided not to attempt to see theoretical noise levels.

## Analysis of Sensor Noise.

- The capacitance of a finite-length, finite-diameter wire:

Cwire = 2*pi*e0*(L/2)/( ln(L/2*a) - 1)   [Jordan, EM Waves and Radiating Systems, 1968]

- For the THEMIS sphere+fine wire (baseline sensor design for RBSP EFW):

- Sphere: 4-cm radius, ~ 4 pF capacitance,

- Fine wire: L = 3 m, 2*a = diameter = 0.23 mm, so:

Cwire = (27.8 pF/m)*L/( 9.48 - 1) = (3.3 pF/m)*(3 m) = 10 pF;

- Total sensor capacitance (to infinity) of 4+10 = 14 pF.

Expected SPB 100m/3m Configuration Sensitivity

Expected AXB 12m/1m Configuration Sensitivity

## Sensitivity versus Noise.

There is a difference between “sensitivity” and “noise”. If one integrates for sufficient periods, one can identify a signal that is ~10 to ~20 dB below the noise level. If IOWA defines sensitivity to be ~10 dB below the noise level then we are done.

## Approaches to Meeting IOWA Noise Requirement

We can reduce the basic sensor noise through one or more of the following options:

- Modify the noise requirement;

- Lengthen the booms.

- Lengthen the fine wire;

- Increase the diameter of the fine wire (reducing the magnitude of the ln(L/D) factor in the denomentator).  This increases the photoemission surface of the fine wire, so that’s probably not a preferred way to go.

- Use the braid of the SPB wire as the IOWA sensor;

- Use (5) with a follower preamplifier.