Laboratory For Atmospheric and Space Physics

LASP Engineering Division

University of Colorado

Boulder, Colorado

Radiation Belt Storm Probes (RBSP) Electric Fields and Waves (EFW) Digital Fields Board (DFB) Specification Document No. RBSP_EFW_DFB_001H_SPEC

| Approvers List |  |
| --- | --- |
| Prepared By | Susan Batiste, Systems Engineer |
| Approved By | Bob Ergun, Co-Investigator |
| Approved By | Magnus Karlsson, DFB FPGA Engineer |
| Approved By | Wes Cole, DFB Analog Electronics Engineer |
| Approved By | Mary Bolton, DFB Program Manager |
| Approved By | Michael Ludlam, EFW Systems Engineer |
| Distribution | John Bonnell, Matt Born, Jorg Fischer, Keith Goetz, Dorothy Gordon, Peter Harvey, Will Rachelson, John Wygant |
| Configuration Management |  |

| Revision History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev | ECO # | Change Description | By | Date |
| 1 |  | DRAFT | S Batiste | 04/16/08 |
| 2 |  | DRAFT | S Batiste | 04/25/08 |
| 3 |  | DRAFT | S Batiste | 05/02/08 |
| 4 |  | DRAFT | S Batiste | 07/02/08 |
| 5 |  | DRAFT | S Batiste | 07/09/08 |
| A | C02162 | Initial Release | S. Batiste | 08/04/08 |
| B | C02413 | Referenced DFB-IDPU MICD in board dimensions Stated explicitly DC signals Updated all APID tables Added MAG backup data Added Command List Section 5.2.1 Added ADC mapping Updated Table 26 through Table 29 Added data compression information, Section 5.3.1 | S. Batiste | 03/16/09 |
| C | C02659 | Updated Figure 1 block diagram. Updated Table 2 and Table 3 to show more detail on the SCM and MAG analog signals from EMFISIS. Updated MAG Survey command defaults in Section 5.2.1.5. Updated E_B2_ENA Burst 2 defaults in Section 5.2.1.7. Added SWD_N_WINDOW parameter to Solitary Wave Detector command. Updated Spectra and Cross Spectra registers Sections Spectra Registers and 5.2.1.11: SPEC_SPD replaced with SPEC_NAVG and SPEC_NCAD, XSPEC_AVG replaced with XSPEC_NAVG and XSPEC_NCAD, Updated compression definitions in Section 5.3.1. Added telemetry packet definitions, Section 5.3.4. Changed Table 22 to list analog signals in the order read out of the MUX by the FPGA, which is also the reporting order from the FPGA. Added the FFT Frequency Band Tables in Section 8.1. Updated data product tables (Table 26, Table 27, Table 28, and Table 29). Removed From/To column in Table 30. | S. Batiste | 05/01/08 |
| D | C02948 | Changed axes on SCM, SC, MAG, FM from 1-2-3 to U-V-W. Changed in text and in block diagram. Changed command description selections from decimal to hexidecimal. Updated Backplane Interface table (Section 9.2) to be consistent with RBSP_EFW_BPL_001L_Specification Updated Solitary Wave Detector registers. Updated limit definitions on solitary wave detector telemetered ranges, Section 5.3.4.3, to create a non-zero lowest limit. | S. Batiste | 07/29/09 |
| E | C03830 | Corrected the ordering of Burst 2 Data Product, Section 5.3.4.6 Corrected bit rate for Survey Waveforms, SWD in Table 26 Corrected Section 5.3.4.6 for Burst 2 (Interferometric), ordering of data in first table Corrected Bins 5 & 6 ordering in Table 11 (FilterBank) | S. Batiste | 01/29/10 |
| F | C04153 | Updated block diagram of DFB with 2 SRAMS. Section 2.1, Fabrication and Test Requirements: Removed TBD on the thermal/vacuum test: will perform thermal test (not vacuum) from -25 to +55. Removed radiated susceptibility testing text; will not be performed. Section 3.1, EFW Analog Sensor Signals: Clarified that CMR of 80 dB and 60 dB (DC and AC signals) are goals, and requirements are 70 and 40 dB at 1 kHz, respectively. Included the gains on the analog signals in table. Sections 3.2, 3.3, SCM and MAG Analog Sensor Signals: Added pointer to Section 9.1 for connector pin-outs. Section 5, Command and Telemetry Interface: Added reference to the IDPU Backplane Specification Section 5.1, Timing: New section, describing the influence of 1 PPS on fast data products and the Super-PPS on slow data products. Section 5.2: Removed note that no command echo by the DFB is expected. Section 5.2.1, Command List: Added definition for the “undefined” values and “unused” bits, and reset values. Changed undefined or “-“ descriptions for address 0x00 bits 15:8, 0x04 bits 15:2, 0x05 bits 15:2 to “unused.” Section 5.2.1.5, Survey Waveforms command definitions: CmdID 0x11, added Bit 6=VDC_AVG and note that the waveform is added for ground testing to be able to test the data product in SPEC and FB. Section 5.2.1.6, Burst #1 Waveforms command definitions: CmdID 0x14, added Bit 6=VDC_AVG and note that the waveform is added for ground testing to be able to test the data product in SPEC and FB. Section 5.2.1.7, Burst #2 Waveforms command definitions: CmdID 0x18, added Bit 5=SCMprp2 and note that the waveform is added for ground testing to be able to test the data product in SPEC and FB. SWD Registers: Address 0x20 updated to include SWD_CHAN_SEL of 3 = Undefined. Address 0x20 presentation of SWD_CHAN_SEL of 2 changed but no change in content or FPGA implementation. Address 0x20 presentation of SWD_RPT_RT changed but no change in content or FPGA implementation Address 0x21-25 reset values added. Section Spectra RegistersSpectra Registers: Reduced from 8 to 7 spectral processors Added note that NCAD ≥ NAVG required for valid data Removed Edcprp2 and Eacprp as source options. SPEC_SRC reordered to account for this change: 0x15-0x18 have been redefined. SPEC_NAVG and SPEC_NCAD selection of 0xB changed from 2048 to Undefined. Removed FPGA capability to average 2048 FFTs or have a 2048 FFT reporting period. Max sampling duration: from 256 to 128 seconds, slowest reporting rate: from 1/256 S/s to 1/128 S/s. Address 0x30 SPEC_NCAD now controls cadence for all XSPEC data products, removed the NCAD command portions of the XSPEC command. Section 5.2.1.10.1, Spectra Default Values: Set default of 0x34, 0x35, 0x36 (SCMW, V1AC, V2AC) to Enabled. Removed 0x37 (SPEC8), capability removed from FPGA. Section 5.2.1.11, Cross-Spectra Registers: Corrected first paragraph to indicate that XSPEC_NAVG for 0x38 used for all other XSPECs (0x39-0x3B). Added note that the address 0x30 SPEC_NCAD now controls cadence for all XSPEC data products. XSPEC_NAVG selection of 0xB changed from 2048 to Undefined. Removed FPGA capability to average 2048 FFTs. Max sampling duration: from 256 to 128 seconds. Removed the NCAD command portions of the XSPEC command. Changed to Unused. Section 5.2.1.10.1, Cross-Spectra Default Values: Removed default values fro XPSEC_NCAD as bits 15:12 now Unused. Corrected typo in description of XSPEC_NAVG for registers 0x39-0x3B: 0x38 value read 4, corrected to 8 FFTs. Section 5.2.1.12.1, Electric Field Registers: Added reset values. Added description of E_B_MATRIX_XX as footnote below table. Section , Magnetic Field Registers: Added reset values. Added description of S_B_MATRIX_XX as footnote below table. Added registers 0x79, 0x7A, 0x7B: MAG offsets. See table for definition and default value. Section 5.2.1.12.3, FAP Configuration Register: Updated command. EAC_OFFSET_SEL now bit 3. Bits 0, 1, 2 are FAP_SCM_ENABLE, FAP_EDC_ENABLE, FAP_EAC_ENABLE. Section 5.2.1.13, DFB Sync with Super PPS: New section. Added text description and command definition (address 0x3F). See section for definition. Section 5.3.1.2, Spectra Compression: No change to compression algorithm. Updated drawing to show two 8-bit compressed values as sent from DFB to DCB. Section 5.3.1.3, Cross Spectra Compression: Multiple edits of text to reflect the reordering of the data product (P, Rc, Ic). Compression of P has not changed. Compression of Rc, Ic changed. No longer uses a spectra exponent; now completely self contained, and signed. Section 5.3.1.4, Solitary Wave Counter Compression: 12-to-8 bit algorithm description added to document. Section 5.3.2, Synchronization: Added Super-PPS description. Changed “preceding (TBR) second” to “that second” Changed “order of APIDs is defined (TBD)” to “order of APIDs is variable”. Reworded requirement for FPGA to send 32 zeros for synch with DCB. Section 5.3.3, APIDs: Corrected typos in the example of FB and V_SVY APIDs and component numbers to transfer. Section 5.3.4, Telemetry Packet Definition: Changed “IDPU processing software” to “EFW FSW” (also added to acronyms table) Added VDC_AVG to V_SVY telemetry packet definition. Added number of data words transferred in SPEC. Removed SPEC8 from FFT telemetry packet. Redefined XSPEC telemetry packet: changes are significant – see Table 9. Added number of data words transferred in XSPEC. Added VDC_AVG to V_B1 telemetry packet definition. Added SCMprp2_B2 to SC_B2 telemetry packet definition. Table 22: Added data product name associated with FM (MAG) and SC (SCM) analog signals. Section 8, Digital Signal Processing: Added note that V1-V4DC average included n real-time waveform signals. Added note that SCprp transformation is in spin plane Added SCprp2 transformation and noted it is out of spin plane. Section 8.1, FFT Frequency Band Tables: No change in bins or frequency ranges. Added note that Frequency is in Hz Added column to show frequency range (dF) in Hz Added color to help distinguish where dF changes in tables. Table 26. Survey data products.: Sample/sec max for waveforms updated to show the difference between what DFB can produce (16K) and what DCB can receive (128). Removed Edcprp2 and Eacprp2 from SPEC source signals. Changed SPEC and XSPEC Sample/sec min from 1/256 to 1/128. Corrected bits/s on MAG_SVY (from 3072 to 1536) Corrected bits/s on SPEC (from 256 to 448) Corrected bits/s on XSPEC (from 512 to 768) Corrected total bits/s (from 9352 to 8264) Table 30, Interface with EMIFIS SCM and MAG, EFW-J703 pinout. Updated “ground” to Analog Ground. All ground signals are analog ground signals. Added Table 31 through Table 36, describing EFW-J715 through EFW-J720 SMA connectors | S. Batiste | 04/09/10 |
| G | C04301 | Section5.2.1.2, ADC Registers: Added operational constraint that ADC1_SRC_SEL≠ ADC2_SRC_SEL. Section 5.2.1.2, ADC Registers and Table 29, SVY_INT: Added note on nominal configuration for ADCn_SRC_SEL and effect on order of SVY_INT data products is source select is swapped. 5.2.1.8,Internal Waveform Command: Changed the SVY_INT_ENA bit assignments. Now corresponds with nominal Mux-ADC read order. Changed ordering E_B2_ENA selections 3-7 for Burst 2 waveforms, register address 0x16. Now corresponds with nominal Mux-ADC read order. Deleted Solitary Wave sections (not implemented). Updated what default values any undefined command parameter defaults to. See Command List for details. Changed command values on startup. FPGA comes up in a quiet state. Exceptions to “0x0” as startup values listed at start of Command List. Added mnemonics to Command List table. | S. Batiste | 05/3/10 |
| H | C05470 | The waveform channels default to a speed of 0x0 (1 S/s) when an invalid command is written to the speed bits (intended to default to 0x5). Register 0x48 should default to 0x7FFF, but is incorrectly set to default to 0x0. FSW should set 0x48 to 0x7FFF at start up. Updated mass to NTE of 504 g (estimate 438 g for ETU) per M. Ludlam email 9/13/10 Added Section 10, describing System Timing. Section 5.1: Removed reference to the Solitary Wave Detector Section 5.2.1.2: Added description for the non-nominal configuration [ADC2_SRC_SEL = 0 and ADC2_SRC_SEL = 1]: spectra and cross spectra will not function properly if any VAC product is selected (data will be noise on all spectra). Spectra and cross spectra will work fine in the non-nominal MUX configuration only so long as all non-VAC sources are selected. Also noted as footnote 5 in Table 26. Section 5.3.1.3: Removed stay empty boxes. Section 5.3.3, Table 4: Changed “interferometric” to “high-rate” on Burst 2 description Section 8: Removed two references in this section to the DFB producing and supplying data on power up. DFB now comes up in quiet state | S. Batiste | 11/03/10 |

## Table of Contents

0.1	Table of Contents	viii

0.2	Table of Tables	ix

0.3	Table of Figures	x

0.4	Applicable Documents	xi

0.5	Reference Documents	xi

0.6	Acronyms/Abbreviations	xi

1	System Overview	1

2	Requirements	2

2.1	Fabrication and Test	2

2.2	Mass and Mechanical Properties	2

2.3	Electrical Power	2

3	Analog Sensor Signals	3

3.1	EFW	3

3.2	SCM	3

3.3	MAG	4

4	Power Interface	4

5	Command and Telemetry Interface	4

5.1	Timing	4

5.2	Command	5

5.2.1	Command List	6

5.2.1.1	Command Processor Registers	6

5.2.1.2	Analog to Digital Converter Registers	7

5.2.1.3	Filter Bank	8

5.2.1.4	Internal Filter Bank	9

5.2.1.5	Survey Waveforms	10

5.2.1.6	Burst #1 Waveforms	11

5.2.1.7	Burst #2 Waveforms	12

5.2.1.8	Internal Waveforms	13

5.2.1.9	Deleted	13

5.2.1.10	Spectra Registers	14

5.2.1.11	Cross-Spectra Registers	17

5.2.1.12	Field Alignment Processor Registers	19

5.2.1.13	DFB Synchronization with Super PPS	21

5.3	Telemetry	22

5.3.1	Compression	22

5.3.1.1	Filter Bank (0x41 and 0x42, Internal and External)	22

5.3.1.2	Spectra	23

5.3.1.3	Cross-Spectra	23

5.3.1.4	Deleted	24

5.3.2	Synchronization	24

5.3.3	APIDs	24

5.3.4	Telemetry Packet Definition	25

5.3.4.1	Waveforms	25

5.3.4.2	FFTs	26

5.3.4.3	Deleted	30

5.3.4.4	Filter Bank	30

5.3.4.5	Burst 1 (Low rate)	31

5.3.4.6	Burst 2 (Interferometric)	32

5.3.4.7	Internal Waveforms	33

5.3.4.8	Internal Filter Bank	33

5.3.4.9	Internal Housekeeping	33

6	Analog to Digital Conversion	33

7	Analog Housekeeping	34

8	Digital Signal Processing	34

8.1	FFT Frequency Band Tables	36

9	Connectors	42

9.1	Detector Interface	42

9.2	Backplane Interface	44

10	DFB FPGA System Timing	45

10.1	Overview	45

10.1.1	Timing Info from DCB	45

10.1.1.1	PPS	46

10.1.1.2	Super-PPS	46

10.1.2	Derived Timing Info	46

10.1.2.1	16KHz Tick	46

10.1.2.2	1/128th Second Bank Switch	47

10.1.3	Data and Embedded Timing Info	47

10.1.4	Data Only	47

10.2	Module Synchronization & Data Path Delay	48

10.2.1	Analog-to-Digital Interface (ADI) Module	48

10.2.1.1	Timing & Synchronization	48

10.2.1.2	Data Path Delay	49

10.2.2	Digital Filters (FIL) Module	50

10.2.2.1	Timing & Synchronization	50

10.2.2.2	Data Path Delay	50

10.2.3	Waveform Player (WAVE) Module	50

10.2.3.1	Timing & Synchronization	51

10.2.3.2	Data Path Delay	51

10.2.4	Triggers (TRIG) Module	52

10.2.4.1	Timing & Synchronization	52

10.2.4.2	Data Path Delay	52

10.2.5	Spectra/X-Spectra (SPEC) Module	54

10.2.5.1	Timing & Synchronization	54

10.2.5.2	Data Path Delay	55

10.2.6	Command Processor	56

10.2.7	Data Processor	57

10.2.7.1	Data Only	57

10.2.7.2	Data and Embedded Timing Info	58

## Table of Tables

Table 1. Analog signals from the EFW to condition for use by EFW.	3

Table 2. Analog signals from the Search Coil Magnetometer to condition.	3

Table 4. Analog signals from the Flux-Gate Magnetometer to condition.	4

Table 4. APID Assignments for data produced by the DFB.	24

Table 5. E_SVY APID 0x43 telemetry packet definition (Bits 23:0)	25

Table 6. V_SVY APID 0x44 telemetry packet definition (Bits 23:0)	25

Table 7. MAG_SVY APID 0x45 telemetry packet definition (Bits 23:0)	25

Table 8. SPEC APID 0x4E telemetry packet definition (Bits 23:0)	26

Table 9. XSPEC APID 0x4F telemetry packet definition (Bits 23:0).	28

Table 10. Deleted	30

Table 11. FB APID 0x41 telemetry packet definition (Bits 23:0) for 7 bins: (FB_FREQBANDS=0).	30

Table 12. FB APID 0x41 telemetry packet definition (Bits 23:0) for 13 bins: (FB_FREQBANDS=1).	31

Table 13. E_B1 APID 0x46 telemetry packet definition (Bits 23:0)	31

Table 14. V_B1 APID 0x47 telemetry packet definition (Bits 23:0)	31

Table 15. SC_B1 APID 0x48 telemetry packet definition (Bits 23:0)	32

Table 16. E_B2 APID 0x49 telemetry packet definition (Bits 23:0)	32

Table 17. V_B2 APID 0x4A telemetry packet definition (Bits 23:0)	32

Table 18. SC_B2 APID 0x4B telemetry packet definition (Bits 23:0)	32

Table 19. E_INT, V_INT, MAG_INT APID 0x4C telemetry packet definition (Bits 23:0)	33

Table 20. FB_INT APID 0x42 telemetry packet definition (Bits 23:0)	33

Table 21. HSKP APID 0x40 telemetry packet definition (Bits 23:0)	33

Table 24. MUX and ADC usage for DFB analog signals. Note sample order alternates between MUX Bank 1 and 2: V1dc, V1ac, V2dc, V2ac, V3dc, V3ac, V4dc, V4ac, V5dc, V5ac, V6dc, V6ac, E12dc, FMU, E34dc, FMV, E56dc, FMW, E12ac, SCU, E34ac, SCV, E56ac, SCW.	34

Table 23. Frequency Table A, 5%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 112 bins are selected.	36

Table 24. Frequency Table B, 10%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 64 bins are selected.	37

Table 25. Frequency Table C, 20%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 36 bins are selected.	38

Table 26. Survey data products.	39

Table 30. Burst 1 (Low Rate) data products.	40

Table 32. Burst 2 (Interferometric) data products.	40

Table 30. Internal data products.	41

Table 35. Interface with EMFISIS SCM and MAG, EFW-J703 pinout.	42

Table 32. Interface with EFW-J715.	42

Table 32. Interface with EFW-J716.	42

Table 33. Interface with EFW-J717.	43

Table 34. Interface with EFW-J718.	43

Table 35. Interface with EFW-J719.	43

Table 38. Interface with EFW-J720.	43

## Table of Figures

Figure 1. Block diagram of DFB.	2

Figure 2. Serial Command Timing	5

Figure 3. DFB Command format.	5

Figure 4. Serial Telemetry Timing	22

Figure 5. DFB data format.	22

Figure 6 DFB Timing Overview	45

## Applicable Documents

| Document Ref | Title |
| --- | --- |
| JHU/APL 7417-9093 | Interface Control Document for the EFW Investigation |
| JHU/APL 7417-9089 | EFW to EMFISIS Electrical Interface Control Document |
| RBSP_EFW_SYS_001 | EFW Requirements Matrix |
| JHU/APL 7417-9019 | RBSP Environmental Design and Test Requirements Document |
| JHU/APL 7314-9018 | RBSP EMC Spacecraft Charging and Magnetics Control Plan |
|  | EFW DFB Compliance Matrix |

## Reference Documents

| Document Ref | Title |
| --- | --- |
|  | EFW DFB FPGA Design and Implementation Document |
| RBSP_EFW_BPL_001 | Backplane Specification |
| RBSP_EFW_BEB_001 | Boom Electronics Board Specification |
| RBSP_EFW_SYS_008 | RBSP EFW Harnessing Diagram |

## Acronyms/Abbreviations

| Acronym | Meaning |
| --- | --- |
| ADC | Analog/Digital Converter |
| BEB | Boom Electronics Board |
| DCB | Data Controller Board |
| DFB | Digital Fields Board |
| EFW | Electric Fields and Waves |
| Epar | Electric field component parallel to the magnetic field |
| Eprp | Electric field component perpendicular to the magnetic field |
| FB | Filter Bank |
| FFT | Fast Fourier Transform |
| FPGA | Field Programmable Gate Array |
| FSW | Flight Software |
| ICD | Interface Control Document |
| IDPU | Instrument Data Processing Unit |
| LASP | Laboratory for Atmospheric and Space Physics |
| LVPS | Low Voltage Power Supply |
| MAG | Flux-Gate Magnetometer |
| MSB | Most-significant bit |
| NC | Not Connected |
| RBSP | Radiation Belt Storm Probes |
| SCM | Search Coil Magnetometer |
| SCMpar | SCM magnetic field component parallel to the magnetic field |
| SCMprp | SCM magnetic field component perpendicular to the magnetic field |
| SMA | Sub-Miniature Type A |
| SRAM | Static Random Access Memory |
| TBD | To be determined |
| TBR | To Be Reviewed |
| UCB | University of California at Berkeley |
| VME | Versa Module Eurocard |

This document contains the design specification for the Digital Fields Board (DFB) that is located in the Electric Fields and Waves (EFW) instrument on the Radiation Belt Storm Probes (RBSP) mission. This document provides a complete description of the design methodology, configuration, functionality and electrical characteristics of the DFB.

In this document, TBD (To Be Determined) indicates that no information currently exists.  TBR (To Be Reviewed) indicates that the information is preliminary.

In the context of this document, statements using the word “shall” are mandatory requirements for the DFB to be verified. Only those requirements stated uniquely within this document make use of the wording “shall”.

Requirements may be imposed by other documents and referred to within this document. In this case, this document does not establish the requirement and uses the wording “is” or “will.” Statements using the words “is” or “will” are descriptive and provide relevant information, but do not positively indicate requirements subject to verification.

Statements using the word “should” indicate goals for which a best effort shall be made. Goals are stated for the purpose of clarifying desired performance and are not to influence the allocation of mission resources.

# System Overview

The Digital Fields Board (DFB) provides analog filtering, analog to digital conversion, and digital signal processing for the EFW instrument sensors as illustrated in Figure 1. The DFB is one board within the EFW Instrument Data Processing Unit (IDPU). The electronics box and remaining boards are provided by University of California at Berkeley (UCB). LASP will design, fabricate, functionally and thermally test, and calibrate the DFB prior to delivery to UCB for integration into the IDPU. Two identical instruments will be built, and LASP will deliver two DFBs, one for each instrument. Each DFB shall be functionally identical [Source: EFW-22]. A flight spare and engineering test unit will also be delivered.

The DFB will receive analog signals from the Electric Fields and Waves (EFW), Flux-Gate Magnetometer (MAG), and Search Coil Magnetometer (SCM).

The EFW measures electric potential at six different points in space using spheres mounted on radial and axial booms to generate signals V1DC-V6DC and V1AC-V6AC. Booms on opposing sides of the spacecraft are paired to create differential electric field measurements E12, E34, and E56 which can be selected as AC (E12AC, E34AC, E56AC) or DC (E12DC, E34DC, E56DC) coupled on the DFB.

The background (DC) magnetic field vector B = [Bu, Bv, Bw] is measured by the MAG and is transferred to the DFB in analog form, with signals MAGU, MAGV, and MAGW. The more rapidly varying (AC) magnetic field vector SCM = [Su, Sv, Sw] is measured by the SCM and is transferred to the DFB in analog form, with signals SCMU, SCMV, and SCMW.

Analog signals will be conditioned, filtered and converted to digital format. The DFB shall provide programmable digital filtering of these signals for transmission as survey data and as burst data. A field programmable gate array (FPGA) will be used to provide spectral analysis on the these signals and report AC and DC electric fields and AC magnetic fields measurements in relation to the DC magnetic field direction based on MAG data. Broadband filter banks of two selected signals will also be available.

Data from the DFB shall be transmitted serially over the Backplane to the Data Controller Board (DCB). The DFB shall receive commands, a 1 PPS and a clock signal from the DCB. The Low Voltage Power Supply(LVPS) shall provide the necessary power to the DFB.

Figure 1. Block diagram of DFB.

# Requirements

Primary EFW requirements can be found in RBSP_EFW_SYS_001(EFW Requirements Matrix).

## Fabrication and Test

The DFB will be fabricated at LASP and shall be in compliance with the EFW DFB Compliance Matrix.

Functionality will be tested and the board will be calibrated and will undergo thermal testing from -25 to +55 ˚C prior to delivery to the instrument for integration.

## Mass and Mechanical Properties

The DFB shall be contained on a single 6U VME printed circuit assembly.  The board shall conform to the mechanical dimensions and clearances as defined in DFB-IDPU Mechanical Interface Control Document (UCB document RBSP-IDP-MEC-200).

The total mass of EFW DFB will not exceed 0.504 g (estimate 0.438 kg) [Source: EFW-103].

## Electrical Power

The EFW DFB shall operate with the following input voltages:

- +/- 10V analog (< 10mV ripple)

- +/- 5V analog (< 10mV ripple)

- +3.6V (<50mV ripple)

- + 1.8V digital (<50mV ripple)

- + 5V digital (<50mV ripple)

# Analog Sensor Signals

The EFW DFB shall receive and condition analog signals from the  EFW, SCM, and MAG [Source: EFW-58]. The EFW DFB shall be designed to filter continuous science measurements [Source: EFW-24]. The DFB will be capable of changing calibration coefficients or tables in response to ground commands [Source: EFW-29]

## EFW

The DFB will accept 6 direct connected probe single ended signals, designated EF1 through EF6, with range of ±225V, from the Boom Electronics Board (BEB) via six Sub-Miniature Type A (SMA) connectors on the front panel.

The signals and conditioning required for the EFW are described below, and in Table 1. All signals will be scaled to ±2.5 V. Common Mode Rejection Ratios of 80 dB (goal, requirement 70 dB) for DC signals and 60 dB (goal, requirement 40 dB) at 1 kHz for AC signals will be implemented. The DFB shall power up in a quiet condition, producing no data products. It is expected the DCB will command the DFB to the nominal flight configuration. Both FPGA reset values (at power up) and nominal (after DCB commanding)  values are provided in Section 8.

  - . Voltage range +/- 100V.

    - Input amplifier gain = 1/20

    - 6 kHz analog low pass filter

    - Attenuation in stop-band ???????

  - 3 differential DC coupled probe signals. Voltage range ????

    - Input amplifier gain = 1/5

    - 6 kHz analog low pass filter

    - Attenuation in stop-band ???????

  - 3 differential AC coupled probe signals. Voltage range +/- 5V.

    - Input amplifier gain = 1

    - 6 kHz analog low pass filter

    - Attenuation in stop-band ???????

  - 1 Logarithmic AC to RMS converter connected to output of 1 differential AC amplifier (E56). 60 dB dynamic range.

Table 1. Analog signals from the EFW to condition for use by EFW.

| Input Signal | Conditioned Signal | Gain Factor | Filtering |
| --- | --- | --- | --- |
| EF1 through EF6 | V1dc through V6dc | 0.01 | low-pass 6.5 kHz |
| EF1 through EF6 | V1ac through V6ac | 0.2 | 10 Hz - 6.5 kHz |
| EF1, EF2 | E12dc | 0.02 | low-pass 6.5 kHz |
| EF3, EF4 | E34dc | 0.02 | low-pass 6.5 kHz |
| EF5, EF6 | E56dc | 0.02 | low-pass 6.5 kHz |
| EF1, EF2 | E12ac | 0.1 | 10 Hz - 6.5 kHz |
| EF3, EF4 | E34ac | 0.1 | 10 Hz - 6.5 kHz |
| EF5, EF6 | E56ac | 0.1 | 10 Hz - 6.5 kHz |

## SCM

The DFB will accept 3 signals, designated SCMU through SCMW. The DFB will accept the signals through a 26 pin high-density D-subminiature connector on the front panel (shared with MAG signals). See Section 9.1 for the pin out. The signals shall have deep dielectric charge tolerance in compliance with the RBSP Electromagnetic Environment Control Plan (APL document 7417-9018).

The signals and conditioning required are described in Table 2.

Table 2. Analog signals from the Search Coil Magnetometer to condition.

| Signal |  | Science Range | Signal Range | Amp Gain | Filtering |
| --- | --- | --- | --- | --- | --- |
| Input | Conditioned |  |  |  |  |
| SCMU, SCMV, SCMW | SCU, SCV, SCW | +/- 5V | +/- 6.8 V | 1/2 differential DC coupled | low-pass 6.5 kHz |

## MAG

The DFB will accept 3 signals, designated MAGU through MAGW. The DFB will accept the signals through a 26 pin high-density D-subminiature connector on the front panel (shared with SCM signals). See Section 9.1 for the pin out. The signals shall have deep dielectric charge tolerance in compliance with the RBSP Electromagnetic Environment Control Plan (APL document 7417-9018). The magnetometer has three gain ranges it may operate on. Upon startup, the FPGA will operate using a default gain state. A CDI command will be sent to the DFB to change the gain.

The signals and conditioning required are described in Table 3.

Table 3. Analog signals from the Flux-Gate Magnetometer to condition.

| Signal |  | Science Range | Signal Range | Amp Gain | Filtering |
| --- | --- | --- | --- | --- | --- |
| Input | Conditioned |  |  |  |  |
| MAGU, MAGV, MAGW | FMU, FMV, FMW | +/- 5V | +/- 6.8 V | 1/2 differential DC coupled | low-pass 6.5 kHz |

# Power Interface

The DFB shall interface to IDPU though the backplane connector. The IDPU will provide +/- 10V analog power for the DFB, +/- 5V analog to power the ADCs, +3.6V digital (to be regulated to +3.3VD on the DFB) for the FPGA and RAM, + 1.8V digital (to be regulated to +1.5VD on the DFB) for the FPGA and +5V digital for analog switch logic. Grounding shall be in accordance to RBSP_EFW_SYS_007_IDPUGrounds, RBSP EFW IDPU Internal Grounding diagram.

# Command and Telemetry Interface

The DFB shall interface to the Digital Control Board (DCB) through the backplane. The DFB shall transmit telemetry parallel through DFB_TLM0 and DFB_TLM1 and receive commands serially from the DCB through the DFB_CMD line. The DCB shall provide the DFB with an 8.389 MHz clock (DFB_CLK) and 1 pps signal (DFB_1HZ). All backplane digital signals shall be +3.3V and shall be defined at the backplane. AC14 drivers will be used to buffer the signals. Further details are available in the IDPU Backplane Specification, RBSP-EFW_BPL_001.

The EFW DFB shall perform instrument command/control functions. The EFW DFB instrument shall reject invalid or unauthorized commands [Source: EFW-96].

The DFB shall produce lower rate as well as scientifically focused data collection (burst) mode [Source: EFW- 45, 46, 48, 49, 51, 52] continuously. Data rates for each data packet type (Table 26-Table 29) are controllable by command, and can be commanded to a rate of zero. DFB shall be capable of operating continuously [Source: EFW-24] producing both normal (survey) and burst mode.

## Timing

The DFB synchronizes telemetry acquisition and reporting to the 1PPS signal. All commands to change configurations will be implemented in the DFB FPGA at the start of the next 1 PPS signal.

For ‘fast’ data products, those that have reporting rates under1 second, data products will not be collected or reported spanning a second boundary as defined by the 1 PPS signal from the DCB. Within the second, the time delay from when the data are first acquired and when they are reported shall be deterministic and will be documented in a timing table.

For ‘slow’ data products, those that have reporting rates longer than 1 second, data are necessarily collected spanning the second boundary as defined by the 1 PPS signal from the DCB. Five quantities can average longer than 1 second, Triggers(FB, FB_INT), Spectra and Cross Spectra. In order for CCSDS packets to start at the beginning of an averaging period requires the DFB and DCB to be in sync. This will be achieved with a “Super-PPS” command to tell the DFB that the next 1PPS signal marks the start of a new averaging cycle (see 5.2.1.13 for command definition). If the DFB receives no signal it will restart its averaging cycle after finishing the last one. The longest averaging cycle is 128 seconds. The DFB will startup and run without the Super-PPS command from the FSW.

## Command

Commands are sent to the DFB from the DCB. Commands are 24-bits wide and are sent synchronously with the rising edge of the DFB_CLK (Figure 2). All signal polarities are as measured on the backplane. The DFB FPGA shall clock in the command data at the falling edge of CLK to avoid race conditions between DFB_CLK and DFB_CMD signals. No command retries will be attempted.

Figure 2. Serial Command Timing

The 8 most significant bits are used for command identification and the remaining 16 bits are command information. The 24-bit commands are sent most significant bit (MSB) first preceded by a start bit and followed by a parity bit then by a stop bit. The parity bit is odd and includes the parity bit and the 24 command bits but not the start bit so that a command with all 24 bits zero would have the parity bit on. Figure 3 illustrates the command format. If the parity bit does not check, an error counter in housekeeping is incremented and the command is ignored.

The DFB synchronizes by finding the first non-zero bit (START bit), and verifies synchronization by the presence of a zero-value STOP bit. After a reset or loss of synchronization, the DFB will look for 25 consecutive zero-level bits before starting to look for a start bit to avoid incorrect interpretation of a transfer in progress.

|  | 23 | … | 18 | 17 | 16 | 15 | … | 2 | 1 | 0 | P |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start bit (1) | 8-bit Command Identification MSB |  |  |  |  | 16-bit Command Information LSB |  |  |  |  | Parity Bit (odd) | Stop Bit (0) |

Figure 3. DFB Command format.

The following sections provide the command list of all possible commands for the DFB FPGA. The first column gives an 8-bit command identification in hex which corresponds to an FPGA address. The following columns detail the 16-bit command information, including a description and start-up default values.

### Command List

This section defines all DFB commands. On start-up, the FPGA comes up in a quiet state. Registers need to be set to the Nominal values (right columns in the following tables) for the FPGA to produce the default flight configuration. All register values are reset to zero on a power cycle, with just a few exceptions:

- The ADC1_SRC_SEL and ADC2_SRC_SEL are assigned to Mux Bank #1 and Mux Bank #2, respectively and enabled. This does not produce any data.

- In the field alignment processor registers (electric field and magnetic field), the 9-element rotation matrices are set to unity matrices, and the gains are set to 1.

- In the FAP register (0x78), the field alignment for SCM signals is enabled. This does not produce any data.

Unused bits: When bits are unused, the FPGA ignores the bits and the value contained in the bits has no effect on the FPGA operation. These instances are shown below as having the name, read/write and reset values of “-“ and description of “Unused”.

Undefined values: When a group of bits in a command are used together to select from a number of options, there are cases where there are fewer options than bit combinations. In these cases, bit combinations are listed as “Undefined.” For example, in the Filter Bank Command (0x06) bits 11:8 define the FB_speed. There are 11 valid reporting speeds, requiring 4 bits to define those speeds, which leaves 16 possible values for the 4 bits. For those 4 bits, values between 0x0 and 0xA are used for the 11 valid reporting speeds and values 0xB through 0xF are “Undefined.” If the FPGA is commanded into an “Undefined” configuration, the FPGA will autonomously revert to a default value, assigned for each data product type. These “defaults to” values are listed in the tables below.

#### Command Processor Registers

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address | Bit | Name/Mneumonic |  |  |  |
| 0x00 | 7:0 | REGISTER_READ | -/W | Writing an address to this register causes a register read packet to be generated containing the contents of that register. | - |
|  | 15:8 | - | - | Unused | - |
| 0x00 | 15:0 | FPGA_REVISION | R/- | Current FPGA revision number. | Current Revision |
| 0x01 | 15:0 | SCRATCHPAD | R/W | General purpose scratchpad register for read/write testing. | 0x0 |
| 0x02 | 15:0 | COMMANDS_ACCEPTED | R/W | Number of commands accepted. | 0x0 |
| 0x03 | 15:0 | COMMANDS_REJECTED | R/W | Number of commands rejected. | 0x0 |

#### Analog to Digital Converter Registers

Non-zero Reset Values:

Upon power-on of the FPGA, the ADC1_SRC_SEL and ADC2_SRC_SEL are assigned to Mux Bank #1 and Mux Bank #2, respectively. Note that 0x04 and 0x05 are not reset to 0x0. The far right column represents both reset and nominal values.

Operational constraints:

- ADC1_SRC_SEL ≠ ADC2_SRC_SEL. This is required regardless of the enable status of either ADC. FPGA will operate but produce bad data if ADCs are set to the same Mux bank.

- For nominal ordering of SVY_INT data products it is necessary that ADC1_SRC_SEL = 0 and ADC2_SRC_SEL=1. See Table 29, Internal Data Products, SVY_INT for nominal and alternate data product ordering.

- In the non-nominal configuration [ADC2_SRC_SEL = 0 and ADC2_SRC_SEL = 1], spectra and cross spectra will not function properly if any VAC product is selected (data will be noise on all spectra).  Spectra and cross spectra will work fine in the non-nominal MUX configuration only so long as all non-VAC sources are selected.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Reset &Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x04 ADC1 | 0 | ADC1_SRC_SEL | R/W | Selects the source for ADC #1: 0 = Mux Bank #1 1 = Mux Bank #2 | 0x0 |
|  | 1 | ADC1_ENA | R/W | Enable for ADC #1: 1 = Enable 0 = Disable | 0x1 |
|  | 15:2 | - | - | Unused | - |
| 0x05 ADC2 | 0 | ADC2_SRC_SEL | R/W | Selects the source for ADC #2: 0 = Mux Bank #1 1 = Mux Bank #2 | 0x1 |
|  | 1 | ADC2_ENA | R/W | Enable for ADC #2: 1 = Enable 0 = Disable | 0x1 |
|  | 15:2 | - | - | Unused | - |

#### Filter Bank

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x06 FB | 3:0 | FB_SEL1 | R/W | Source selects for the Filter Bank: 0=E12DC, 1=E34DC, 2=E56DC, 3=E12AC, 4=E34AC, 5=E56AC, 6=SCMU, 7=SCMV, 8=SCMW, 9=(V1DC+V2DC+V3DC+V4DC)/4 0xA-0xF=Undefined (defaults to 0x0) | 0x0 |
|  | 7:4 | FB_SEL2 | R/W |  | 0x0 |
|  | 11:8 | FB_SPD | R/W | Selects the reporting speed for the Filter Bank: 0=1/16S/s, 1=1/8S/s, 2=1/4S/s, 3=1/2S/s, 4=1S/s, 5=2S/s, 6=4S/s, 7=8S/s, 8=16S/s, 9=32S/s, 0xA=64S/s, 0xB-0xF=Undefined (defaults to 0x7) | 0x7 |
|  | 12 | FB_ENA1 | R/W | Enable for the Filter Bank: 1=Enable 0=Disable | 0x1 |
|  | 13 | FB_ENA2 | R/W |  | 0x0 |
|  | 14 | FB_FREQBANDS | R/W | Frequency band selection for the Filter Bank: 0 = 7 bins: [0.8-1.5, 3-6, 12-25, 50-100, 200-400, 800-1.6k, 3.2k-6.5k] 1 = 13 bins: [0.8-1.5, 1.5-3, 3-6, 6-12, 12-25, 25-50, 50-100, 100-200, 200-400, 400-800, 800-1.6k, 1.6k-3.2k, 3.2k-6.5k] | 0x0 |
|  | 15 | - | - | Unused | - |

#### Internal Filter Bank

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x07 FB_INT | 3:0 | FB_INT_SEL3 | R/W | Source selects for the Internal Filter Bank: 0=E12DC, 1=E34DC, 2=E56DC, 3=E12AC, 4=E34AC, 5=E56AC, 6=SCMU, 7=SCMV, 8=SCMW, 9=(V1DC+V2DC+V3DC+V4DC)/4 0xA-0xF=Undefined (defaults to 0x0) | 0x0 |
|  | 7:4 | FB_INT_SEL4 | R/W |  | 0x0 |
|  | 11:8 | FB_INT_SPD | R/W | Selects the reporting speed for the Internal Filter Bank: 0=1/16S/s, 1=1/8S/s, 2=1/4S/s, 3=1/2S/s, 4=1S/s, 5=2S/s, 6=4S/s, 7=8S/s, 8=16S/s, 9=32S/s, 0xA=64S/s, 0xB-0xF=Undefined (defaults to 0x9) | 0x9 |
|  | 12 | FB_INT_ENA3 | R/W | Enable for the Internal Filter Bank: 1=Enable 0=Disable | 0x1 |
|  | 13 | FB_INT_ENA4 | R/W |  | 0x0 |
|  | 14 | FB_INT_FREQBANDS | R/W | Frequency band selection for the Internal Filter Bank: 0 = 7 bins: [0.8-1.5, 3-6, 12-25, 50-100, 200-400, 800-1.6k, 3.2k-6.5k] 1 = 13 bins: [0.8-1.5, 1.5-3, 3-6, 6-12, 12-25, 25-50, 50-100, 100-200, 200-400, 400-800, 800-1.6k, 1.6k-3.2k, 3.2k-6.5k] | 0x1 |
|  | 15 | - | - | Unused | - |

#### Survey Waveforms

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x10 E_SVY | 2:0 | E_SVY_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=E12DC, 1=E34DC, 2=E56DC | 0x7 |
|  | 11:3 | - | - | Unused | - |
|  | 15:12 | E_SVY_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x5 |
| 0x11 V_SVY | 6:0 | V_SVY_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=V1DC, 1=V2DC, 2=V3DC, 3=V4DC, 4=V5DC, 5=V6DC, 6=VDC_AVG* | 0x3F |
|  | 11:7 | - | - | Unused | - |
|  | 15:12 | V_SVY_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x5 |
| 0x12 MAG | 2:0 | MAG_SVY_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=MAGU, 1=MAGV, 2=MAGW | 0x7 |
|  | 11:3 | - | - | Unused | - |
|  | 15:12 | MAG_SVY_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x5 |

*VDC_AVG, (V1dc+V2dc+V3dc+V4dc)/4, implemented for ground testing. Not required in-flight.

#### Burst #1 Waveforms

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Addres/ Mnemonic | Bit | Name |  |  |  |
| 0x13 E_B1 | 2:0 | E_B1_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=E12DC, 1=E34DC, 3=E56DC | 0x7 |
|  | 11:3 | - | - | Unused | - |
|  | 15:12 | E_B1_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x9 |
| 0x14 V_B1 | 6:0 | V_B1_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=V1DC, 1=V2DC, 2=V3DC, 3=V4DC, 4=V5DC, 5=V6DC, 6=VDC_AVG* | 0x3F |
|  | 11:7 | - | - | Unused | - |
|  | 15:12 | V_B1_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x9 |
| 0x15 SCM_B1 | 2:0 | SCM_B1_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=SCMU, 1=SCMV, 2=SCMW | 0x7 |
|  | 11:3 | - | - | Unused | - |
|  | 15:12 | SCM_B1_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x9 |

*VDC_AVG, (V1dc+V2dc+V3dc+V4dc)/4,  implemented for ground testing. Not required in-flight.

#### Burst #2 Waveforms

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x16 E_B2 | 9:0 | E_B2_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=E12DC, 1=E34DC, 2=E56DC, 3=E12AC, 4=E34AC, 5=E56AC, 6=EDCpar, 7=EDCprp, 8=EACpar, 9=EACprp | 0x38 |
|  | 11:10 | - | - | Unused | - |
|  | 15:12 | E_B2_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0xE |
| 0x17 V_B2 | 5:0 | V_B2_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=V1AC, 1=V2AC, 2=V3AC, 3=V4AC, 4=V5AC, 5=V6AC | 0x3F |
|  | 11:6 | - | - | Unused | - |
|  | 15:12 | V_B2_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0xE |
| 0x18 SCM_B2 | 5:0 | SCM_B2_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=SCMU, 1=SCMV, 2=SCMW, 3=SCMpar, 4=SCMprp, 5=SCMprp2* | 0x7 |
|  | 11:6 | - | - | Unused | - |
|  | 15:12 | SCM_B2_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0xE |

*SCMprp2 implemented for ground testing. Not required in-flight.

#### Internal Waveforms

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x19 SVY_INT | 11:0 | SVY_INT_ENA | R/W | Waveform enable: 1=Enable, 0=Disable Bits: 0=V1DC, 1=V2DC, 2=V3DC, 3=V4DC, 4=V5DC, 5=V6DC, 6=E12DC, 7=MAGU, 8=E34DC, 9=MAGV, 10=E56DC, 11=MAGW | 0xFFF |
|  | 15:12 | SVY_INT_SPD | R/W | Speed select: 0=1 S/s, 1=2 S/s, 2=4 S/s, 3=8 S/s, 4=16 S/s, 5=32 S/s, 6=64 S/s, 7=128 S/s, 8=256 S/s, 9=512 S/s, 0xA=1024 S/s, 0xB=2048 S/s, 0xC=4096 S/s, 0xD=8192 S/s, 0xE=16384 S/s, 0xF=Undefined (defaults to 0x0) | 0x5 |

#### Deleted

#### Spectra Registers

The DFB FPGA provides seven spectral processors, each of which is configured via its own register. SPEC1 is configured by address 0x30, SPEC2 with 0x31, and so forth, through SPEC7 with address 0x36. Following is a description of the configuration register for a single spectral processor.  The register format for the other processors is similar except for the address offset and the fact that a single frequency binning configuration, averaging quantity, and cadence is allowed for all of the processors.

Note: SPEC 1 (0x30) must be configured such that NCAD ≥ NAVG in order to have valid science. When NAVG < NCAD, the averaged FFTs will be the number selected taken at the start of the cadence period. Once the number selected have been collected, FFT collection stops until the start of the next collection period. In addition, NAVGspec ≥ NAVGxspec for valid data. In summary:

For valid data, the following is required: NCAD ≥ NAVGspec ≥ NAVGxspec, whether or not XSPEC is enabled.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/Mnemonic | Bit | Name |  |  |  |
| 0x30-0x36 0X30 SPEC1 0X31 SPEC2 0X32 SPEC3 0X33 SPEC4 0X34 SPEC5 0X35 SPEC6 0X36 SPEC7 | 4:0 | SPEC_SRC | R/W | Source select: 0=E12dc, 1=E34dc, 2=E56dc, 3=E12ac, 4=E34ac, 5=E56ac, 6=Edcpar, 7=Edcprp, 8=Eacpar, 9=Eacprp, 0xA=V1ac, 0xB=V2ac, 0xC=V3ac, 0xD=V4ac, 0xE=V5ac, 0xF=V6ac, 0x10=SCMU, 0x11=SCMV, 0x12=SCMW, 0x13=SCMpar, 0x14=SCMprp, 0x15=SCMprp2, 0x16=(V1dc+V2dc+V3dc+V4dc)/4, 0x17-0x18=Undefined (defaults to 0x3 for 0x30, 0x12 for 0x31-36) | See 5.2.1.10.1. |
|  | 5 | SPEC_ENA | R/W | Spectra enable: 1=Enable, 0=Disable | See 5.2.1.10.1 |
|  | 7:6 | SPEC_BIN | R/W | Selects the number of frequency bins inserted into the telemetry stream: 0=36, 1=64, 2=112, 3 = Undefined (defaults to 0x1) Note: this field is only valid for address 0x30 which controls the frequency bins for all spectral processors. | See 5.2.1.10.1 |
|  | 11:8 | SPEC_NAVG | R/W | Number of 2048-point FFTs to average in each period defined by NCAD1: 0=1, 1=2, 2=4, 3=8, 4=16, 5=32, 6=64, 7=128, 8=256, 9=512, 0xA=1024, 0xB-0xF=Undefined (defaults to 0x3) Note: this field is only valid for address 0x30 which controls averaging for all spectral processors. | See 5.2.1.10.1 |
|  | 15:12 | SPEC_NCAD | R/W | Number of 2048-point FFTs in a reporting period (defines cadence of transmission)2: 0=1, 1=2, 2=4, 3=8, 4=16, 5=32, 6=64, 7=128, 8=256, 9=512, 0XA=1024, 0xB-0xF =Undefined (defaults to 0x6) Note: this field is only valid for address 0x30 which controls the cadence for all spectral and cross-spectral processors. | See 5.2.1.10.1 |

1To find SPEC_NAVG, sampling duration, in seconds: using a 16,384Hz sampling rate and 2048-pt FFTs, there are 8 FFTs per second: Selected Value8 FFT/sec=Sample Duration, sec,

e.g. SPEC_NAVG=3: 8 FFT8 FFT/sec=1sec.

| Selection | # FFTs to Average | Sampling duration, sec |
| --- | --- | --- |
| 0 | 1 | 1/8 |
| 1 | 2 | 1/4 |
| 2 | 4 | 1/2 |
| 3 | 8 | 1 |
| 4 | 16 | 2 |
| 5 | 32 | 4 |
| 6 | 64 | 8 |
| 7 | 128 | 16 |
| 8 | 256 | 32 |
| 9 | 512 | 64 |
| 0xA | 1024 | 128 |

2For reporting period (SPEC_NCAD)  in samples/second:8 FFT/secSelected Value=Samplessec,

e.g. for SPEC_NCAD=6: 8 FFT/sec64 FFT/Sample=1 8Samplesec.

| Selection | # FFTs in a reporting period | Reporting rate, samples/sec |
| --- | --- | --- |
| 0 | 1 | 8 |
| 1 | 2 | 4 |
| 2 | 4 | 2 |
| 3 | 8 | 1 |
| 4 | 16 | 1/2 |
| 5 | 32 | 1/4 |
| 6 | 64 | 1/8 |
| 7 | 128 | 1/16 |
| 8 | 256 | 1/32 |
| 9 | 512 | 1/64 |
| 0xA | 1024 | 1/128 |

##### Spectra Nominal Values

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x30 SPEC1 | 4:0 | SPEC_SRC | R/W | SPEC1: E12AC | 0x3 |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Report 64 frequency bins | 0x1 |
|  | 11:8 | SPEC_NAVG | R/W | Average 8 FFTs per period. | 0x3 |
|  | 15:12 | SPEC_NCAD | R/W | Report every 64 FFTs (8 sec). | 0x6 |
| 0x31 SPEC2 | 4:0 | SPEC_SRC | R/W | SPEC2: E56AC | 0x5 |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |
| 0x32 SPEC3 | 4:0 | SPEC_SRC | R/W | SPEC3: SCMpar | 0x13 |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |
| 0x33 SPEC4 | 4:0 | SPEC_SRC | R/W | SPEC4: SCMprp | 0x14 |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |
| 0x34 SPEC5 | 4:0 | SPEC_SRC | R/W | SPEC5: SCMW | 0x12 |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |
| 0x35 SPEC6 | 4:0 | SPEC_SRC | R/W | SPEC6: V1AC | 0xA |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |
| 0x36 SPEC7 | 4:0 | SPEC_SRC | R/W | SPEC7: V2AC | 0xB |
|  | 5 | SPEC_ENA | R/W | Enabled | 0x1 |
|  | 7:6 | SPEC_BIN | R/W | Uses address 0x30 value (64 freq bins) | N/A |
|  | 11:8 | SPEC_NAVG | R/W | Uses address 0x30 value (8 FFTs) | N/A |
|  | 15:12 | SPEC_NCAD | R/W | Uses address 0x30 value (64 FFTs) | N/A |

#### Cross-Spectra Registers

The DFB FPGA provides four cross-spectral processors, each of which is configured via its own register.  Following is a description of the configuration register for a single cross-spectral processor.  The register format for the other processors is similar except for the address offset and the fact that a single number of FFTs to average per cross-spectra is defined by 0x38.

For all cross-spectra, the period cadence is defined by command 0x30 (SPEC1).

As indicated in the SPEC section previous, for valid data, the following is required: NCAD ≥ NAVGspec ≥ NAVGxspec, whether or not XSPEC is enabled.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x38-0x3B 0X38 XSPEC1 0X39 XSPEC2 0X3A XSPEC3 0X3B XSPEC4 | 2:0 | XSPEC_SRC1 | R/W | Source select1: 0=SPEC1, 1=SPEC2, 2=SPEC3, 3=SPEC4, 4=SPEC5, 5=SPEC6, 6=SPEC7, 7=Undefined (defaults to 0x4, SPEC5 for 0x38, 0x5, SPEC6, for 0x39-3B) | See 5.2.1.11.1. |
|  | 5:3 | XSPEC_SRC2 | R/W | Source select1: 0=SPEC1, 1=SPEC2, 2=SPEC3, 3=SPEC4, 4=SPEC5, 5=SPEC6, 6=SPEC7, 7= Undefined (defaults to 0x0, SPEC1 for 0x38, 0x6, SPEC7, for 0x39-3B) | See 5.2.1.11.1. |
|  | 6 | XSPEC_ENA | R/W | Spectra enable: 1=Enable, 0=Disable | See 5.2.1.11.1. |
|  | 7 | - | - | Unused | - |
|  | 11:8 | XSPEC_NAVG | R/W | Number of 2048-point FFTs to average in each period defined by NCAD2,3: 0=1, 1=2, 2=4, 3=8, 4=16, 5=32, 6=64, 7=128, 8=256, 9=512, 0xA=1024, 0xB-0xF =Undefined (defaults to 0x3) This field is only valid for address 0x38 which controls averaging for all cross-spectral processors. | See 5.2.1.11.1. |
|  | 15:12 | - | - | Unused | - |

1 XSPEC_SRC1 and XSPEC_SRC2 are SPEC quantities defined by addresses 0x30-0x37:

SPEC1 = SPEC configured with address 0x30	SPEC2 = SPEC configured with address 0x31

SPEC3 = SPEC configured with address 0x32	SPEC4 = SPEC configured with address 0x33

SPEC5 = SPEC configured with address 0x34	SPEC6 = SPEC configured with address 0x35

SPEC7 = SPEC configured with address 0x36	SPEC8 = SPEC configured with address 0x37

2 To find sampling duration in seconds: using a 16,384Hz sampling rate and 2048-pt FFTs, there are 8 FFTs per second: Selected Value8 FFT/sec=Sample Duration, sec, e.g. XSPEC_NAVG=3: 8 FFT8 FFT/sec=1sec.

3 For valid data, must set XSPEC_NAVG ≤ SPEC_NAVG

##### Cross-Spectra Nominal Values

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x38 XSPEC1 | 2:0 | XSPEC_SRC1 | R/W | SCMW (SPEC5) | 0x4 |
|  | 5:3 | XSPEC_SRC2 | R/W | E12AC (SPEC1) | 0x0 |
|  | 6 | XSPEC_ENA | R/W | Enabled | 0x1 |
|  | 7 | - | - | Unused | - |
|  | 11:8 | XSPEC_NAVG | R/W | Average 8 FFTs per period. | 0x3 |
|  | 15:12 | - | - | Unused | - |
| 0x39 XSPEC2 | 2:0 | XSPEC_SRC1 | R/W | V1AC (SPEC6) | 0x5 |
|  | 5:3 | XSPEC_SRC2 | R/W | V2AC (SPEC7) | 0x6 |
|  | 6 | XSPEC_ENA | R/W | Enabled | 0x1 |
|  | 7 | - | - | Unused | - |
|  | 11:8 | XSPEC_NAVG | R/W | Uses address 0x38 value (8 FFTs) | N/A |
|  | 15:12 | - | - | Unused | - |
| 0x3A XSPEC3 | 2:0 | XSPEC_SRC1 | R/W | none selected | 0x0 |
|  | 5:3 | XSPEC_SRC2 | R/W | none selected | 0x0 |
|  | 6 | XSPEC_ENA | R/W | Disabled | 0x0 |
|  | 7 | - | - | Unused | - |
|  | 11:8 | XSPEC_NAVG | R/W | Uses address 0x38 value (8 FFTs) | N/A |
|  | 15:12 | - | - | Unused | - |
| 0x3B XSPEC4 | 2:0 | XSPEC_SRC1 | R/W | none selected | 0x0 |
|  | 5:3 | XSPEC_SRC2 | R/W | none selected | 0x0 |
|  | 6 | XSPEC_ENA | R/W | Disabled | 0x0 |
|  | 7 | - | - | Unused | - |
|  | 11:8 | XSPEC_NAVG | R/W | Uses address 0x38 value (8 FFTs) | N/A |
|  | 15:12 | - | - | Unused | - |

#### Field Alignment Processor Registers

##### Electric Field Registers

Note that registers 0x40, 0x44, 0x48, 0x54, 0x55, 0x56, 0x5C, 0x5D, 0x5E are not reset to 0 at power up in order to produce a unity matrix for E_B matrix and a unity gain for the EDC and EAC gains. The far right column represents both reset and nominal values.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Reset& Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address | Bit | Name/Mnemonic |  |  |  |
| 0x40 | 15:0 | E_B_MATRIX_11[15:0] | R/W | 9-element rotation matrix to align the B coordinate system to the E coordinate system. Elements are labeled with subscripts ij where i=row, j=col. - twos complement representation E B_MATRIX_xx range = -1 to 1* | 0x7FFF |
| 0x41 | 15:0 | E_B_MATRIX_12[15:0] | R/W |  | 0 |
| 0x42 | 15:0 | E_B_MATRIX_13[15:0] | R/W |  | 0 |
| 0x43 | 15:0 | E_B_MATRIX_21[15:0] | R/W |  | 0 |
| 0x44 | 15:0 | E_B_MATRIX_22[15:0] | R/W |  | 0x7FFF |
| 0x45 | 15:0 | E_B_MATRIX_23[15:0] | R/W |  | 0 |
| 0x46 | 15:0 | E_B_MATRIX_31[15:0] | R/W |  | 0 |
| 0x47 | 15:0 | E_B_MATRIX_32[15:0] | R/W |  | 0 |
| 0x48 | 15:0 | E_B_MATRIX_33[15:0] | R/W |  | 0x0 (reset) 0x7FFF (nominal) |
| 0x50 | 15:0 | EDC_X_OFFSET[15:0] | R/W | Offsets for DC-coupled E field data: - twos complement representation - offsets are subtracted from E | 0 |
| 0x51 | 15:0 | EDC_Y_OFFSET[15:0] | R/W |  | 0 |
| 0x52 | 15:0 | EDC_Z_OFFSET[15:0] | R/W |  | 0 |
| 0x54 | 15:0 | EDC_X_GAIN[15:0] | R/W | Gains for DC-coupled E field data: - twos complement representation - gain range = -1 to 1 | 0x7FFF |
| 0x55 | 15:0 | EDC_Y_GAIN[15:0] | R/W |  | 0x7FFF |
| 0x56 | 15:0 | EDC_Z_GAIN[15:0] | R/W |  | 0x7FFF |
| 0x58 | 15:0 | EAC_X_OFFSET[15:0] | R/W | Offsets for AC-coupled E field data: - twos complement representation - offsets are subtracted from E. | 0 |
| 0x59 | 15:0 | EAC_Y_OFFSET[15:0] | R/W |  | 0 |
| 0x5A | 15:0 | EAC_Z_OFFSET[15:0] | R/W |  | 0 |
| 0x5C | 15:0 | EAC_X_GAIN[15:0] | R/W | Gains for AC-coupled E field data: - twos complement representation - gain range = -1 to 1 | 0x7FFF |
| 0x5D | 15:0 | EAC_Y_GAIN[15:0] | R/W |  | 0x7FFF |
| 0x5E | 15:0 | EAC_Z_GAIN[15:0] | R/W |  | 0x7FFF |

* E_B_MATRIX_XX is a scaled two's complement number where 215= -1 and 215-1 = 1.

Numbers between 215 and 215-1 scale linearly between -1 and 1.

##### Magnetic Field Registers

Note that registers 0x60, 0x64, 0x68, 0x74, 0x75 and 0x76 are not reset to 0 at power up in order to produce a unity matrix for S_B matrix and a unity gain for the S gains. The far right column represents both reset and nominal values.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Reset &Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address | Bit | Name/Mnemonic |  |  |  |
| 0x60 | 15:0 | S_B_MATRIX_11[15:0] | R/W | 9-element rotation matrix to align the B coordinate system to the S coordinate system. Elements are labeled with subscripts ij where i=row, j=col. - twos complement representation - S_B_MATRIX_xx range = -1 to 1* | 0x7FFF |
| 0x61 | 15:0 | S_B_MATRIX_12[15:0] | R/W |  | 0 |
| 0x62 | 15:0 | S_B_MATRIX_13[15:0] | R/W |  | 0 |
| 0x63 | 15:0 | S_B_MATRIX_21[15:0] | R/W |  | 0 |
| 0x64 | 15:0 | S_B_MATRIX_22[15:0] | R/W |  | 0x7FFF |
| 0x65 | 15:0 | S_B_MATRIX_23[15:0] | R/W |  | 0 |
| 0x66 | 15:0 | S_B_MATRIX_31[15:0] | R/W |  | 0 |
| 0x67 | 15:0 | S_B_MATRIX_32[15:0] | R/W |  | 0 |
| 0x68 | 15:0 | S_B_MATRIX_33[15:0] | R/W |  | 0x7FFF |
| 0x70 | 15:0 | S_X_OFFSET[15:0] | R/W | Offsets for S data: - twos complement representation - offsets are subtracted from S. | 0 |
| 0x71 | 15:0 | S_Y_OFFSET[15:0] | R/W |  | 0 |
| 0x72 | 15:0 | S_Z_OFFSET[15:0] | R/W |  | 0 |
| 0x74 | 15:0 | S_X_GAIN[15:0] | R/W | Gains for S data: - twos complement representation - gain range = -1 to 1 | 0x7FFF |
| 0x75 | 15:0 | S_Y_GAIN[15:0] | R/W |  | 0x7FFF |
| 0x76 | 15:0 | S_Z_GAIN[15:0] | R/W |  | 0x7FFF |
| 0x79 | 15:0 | B_X_OFFSET[15:0] | R/W | Offsets for MAG data: - twos complement representation - offsets are subtracted from S. | 0 |
| 0x7A | 15:0 | B_Y_OFFSET[15:0] | R/W |  | 0 |
| 0x7B | 15:0 | B_Z_OFFSET[15:0] | R/W |  | 0 |

* S_B_MATRIX_XX is a scaled two's complement number where 215= -1 and 215-1 = 1. Numbers between 215 and 215-1 scale linearly between -1 and 1.

##### Field Alignment Processor Configuration Register

Note that register 0x78, FAP, is not reset to 0 at power up. Instead, the field alignment for SCM signals is enabled. The far right column represents both reset and nominal values.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Reset& Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address/ Mnemonic | Bit | Name |  |  |  |
| 0x78 FAP | 0 | FAP_SCM_ENABLE | R/W | 1 = enable field alignment for SCM signals | 0x1 |
|  | 1 | FAP_EDC_ENABLE | R/W | 1 = enable field alignment for DC coupled E signals | 0x0 |
|  | 2 | FAP_EAC_ENABLE | R/W | 1 = enable field alignment for AC coupled E signals | 0x0 |
|  | 3 | EAC_OFFSET_SEL | R/W | 0 = Use commanded DC offsets for EAC offsets 1 = Use calculated (low-pass) DC offsets for EAC offsets | 0x0 |
|  | 15:4 | - | - | Unused | 0x0 |

#### DFB Synchronization with Super PPS

The following command is used to synchronize data products whose rate spans more than 1 second. The DFB FPGA will look for write access on Register 0x3F and act on the write access rather than any particular value written to the register. When the DFB FPGA sees that write access, it will use the next 1-PPS as a Super-PPS and resync all data products.

The Super PPS should occur at a rate equal or greater to all the reporting periods for SPEC, XSPEC, Triggers (FB and FB_INT), and SWD to allow time for these data products to complete processing.

| 8-bit Cmd ID | 16-bit Command Information |  | Read/ Write | Description | 16-bit Cmd Information Nominal Value |
| --- | --- | --- | --- | --- | --- |
| Address | Bit | Name/Mnemonic |  |  |  |
| 0x3F | 15:0 | SUPER_PPS | R/W | When register is written to, the following 1 PPS will also be a Super-PPS. Read of address returns 0. | - |

## Telemetry

Data are sent to the DCB from the DFB. Data packets are 24-bits wide and are sent synchronously with the rising edge of the DFB_CLK. All signal polarities are measured on the backplane. Telemetry data is transmitted by the DFB at various rates depending on operational modes and configurations, up to a theoretical maximum of 512 kwords/sec. Data are transmitted in parallel over TLM_0 and TLM_1. To avoid any data sequence ambiguity, transmissions operate on 32-bit slots, with the transmissions of TLM_0 and TLM_1 offset by 16 clocks. This guarantees that the DFB shall never transmit the same APID on the two TLM-lines simultaneously and produces a very deterministic data stream.

Figure 4. Serial Telemetry Timing

The 8 most significant bits are the data identification (DATA_ID) as defined in the tables below, and the remaining16-bits are the value of the variable indicated by the DATA_ID.  The 24-bit data are sent MSB first preceded by a start bit and followed by a parity bit then by a stop bit. The parity bit is odd and includes the parity bit and the 24 data bits but not the start bit so that a data packet with all 24 bits zero would have the parity bit on.  Figure 5 illustrates the telemetry data format.

|  | 23 | … | 18 | 17 | 16 | 15 | … | 2 | 1 | 0 | P |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start bit (1) | 8-bit Data Identification MSB |  |  |  |  | 16-bit Data Value LSB |  |  |  |  | Parity Bit (odd) | Stop Bit (0) |

Figure 5. DFB data format.

### Compression

The DFB performs compression on Filter Bank, Spectra and Cross-Spectra prior to telemetering data. Each are described below.

#### Filter Bank (0x41 and 0x42, Internal and External)

The Filter Bank (APID 0x41) telemetry uses a 19-to-8 bit compression algorithm:

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E | E | E | E | M | M | M | M |
|  | Exponent |  |  |  | Mantissa |  |  |  |

When the exponent (EEEE) is zero, the uncompressed value is simply the mantissa, MMMM.

When the exponent (EEEE) is non-zero, an upper bit is added to the mantissa (mantissa = 1MMMM) and the uncompressed value is found through the following equation:

Value= 1MMMM x 2EEEE-1

#### Spectra

The Spectra (APID 0x4E) telemetry uses a 34-to-8 bit compression algorithm. Shown here are two 8-bit compressed values, concatenated as is reported to the DCB (see FFT packet format, section 5.3.4.2):

|  | Bin N+1: P = R2 + I2 |  |  |  |  |  |  |  | Bin N: P = R2 + I2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | E | E | E | E | E | M | M | M | E | E | E | E | E | M | M | M |
|  | Exponent |  |  |  |  | Mantissa |  |  | Exponent |  |  |  |  | Mantissa |  |  |

When the exponent (EEEEE) is zero, the uncompressed value is simply the mantissa, MMM.

When the exponent (EEEEE) is non-zero, an upper bit is added to the mantissa (mantissa = 1MMM) and the uncompressed value is found through the following equation:

Value= 1MMM x 2EEEEE-1

#### Cross-Spectra

The Cross-Spectra (APID 0x4F) telemetry uses a combination of compression algorithms.

A single cross-spectra is transmitted in 3 groups of data packets. The first and second data packet groups  contain spectra data on for each of the cross-spectra components, PXSPEC_SRC1 (R2XSPEC_SRC1 + I2XSPEC_SRC1) and PXSPEC_SRC2 (R2XSPEC_SRC2 + I2XSPEC_SRC2), respectively.

The third data packet group contains Rc (RXSPEC_SRC1RXSPEC_SRC2 + IXSPEC_SRC1IXSPEC_SRC2) and Ic (RXSPEC_SRC1IXSPEC_SRC2 -RXSPEC_SRC2IXSPEC_SRC1), with the compression algorithm described below. Each Rc and Ic are 16 bits, and are ordered as shown in Table 9 where Bin 0 Rc is followed by Bin 0 Ic, then Bin 1 Rc followed by Bin 1 Rc, and so forth through Bin N Rc followed by Bin N Ic.

The first and second data packet set use the same compression algorithm as the spectra data. Shown here are two 8-bit compressed values, concatenated as is reported to the DCB (see FFT packet format, section 5.3.4.2)::

|  | Bin N+1: PXSPEC_SRC1 = R2XSPEC_SRC1 + I2XSPEC_SRC1 |  |  |  |  |  |  |  | Bin N: PXSPEC_SRC1 = R2XSPEC_SRC1 + I2XSPEC_SRC1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | E | E | E | E | E | M | M | M | E | E | E | E | E | M | M | M |
|  | Exponent |  |  |  |  | Mantissa |  |  | Exponent |  |  |  |  | Mantissa |  |  |

For each datum, when its exponent (EEEEE) is zero, the uncompressed value is simply its mantissa, MMM.

For each datum, when its exponent (EEEEE) is non-zero, an upper bit is added to its mantissa (mantissa = 1MMM) and its uncompressed value is found through the following equation:

Value= 1MMM x 2EEEEE-1

The third data packet set uses an algorithms similar to previously presented algorithms, with exception to the sign bit that is present as Cross-Spectra are signed values:

|  | Rc RXSPEC_SRC1RXSPEC_SRC2 + IXSPEC_SRC1IXSPEC_SRC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | S | E | E | E | E | E | M | M | M | M | M | M | M | M | M | M |
|  | Sign | Exponent |  |  |  |  | Mantissa |  |  |  |  |  |  |  |  |  |

|  | Ic RXSPEC_SRC1IXSPEC_SRC2 -RXSPEC_SRC2IXSPEC_SRC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | S | E | E | E | E | E | M | M | M | M | M | M | M | M | M | M |
|  | Sign | Exponent |  |  |  |  | Mantissa |  |  |  |  |  |  |  |  |  |

For each datum, when its exponent (EEEEE) is zero, the uncompressed value is simply its mantissa, MMMMMMMMMM.

For each datum, when its exponent (EEEEE) is non-zero, an upper bit is added to its mantissa (mantissa = 1MMMMMMMMMM) and its uncompressed value is found through the following equation:

Value= S1MMMMMMMMMM x 2EEEEE-1

When S=0, Value is positive. When S=1, Value is negative.

#### Deleted

### Synchronization

The DFB will synch to the 1 PPS and Super-PPS signals from the DCB. Following the 1 PPS signal, the DFB will start at the beginning of the data product series and proceed to telemeter data products for the that second. The order of the APIDs is variable. The Super-PPS is used to resync the processing of the data products where a single product spans over one second. When configured, the processing of the data products begin on the next 1 PPS and the Super-PPS may be used to resync.

To maintain synchronization between the DFB and the DCB, the DFB must drive the telemetry lines low for 32 consecutive clock cycles at least once every second. Nominally, the DCB synchronizes both telemetry streams independently by finding the first non-zero bit (START bit) and verifies synchronization by the presence of a zero-value STOP bit. After a reset or loss of synchronization, the DCB can look for 25 consecutive zero-level bits before starting to look for a start bit to avoid incorrect interpretation of a transfer in progress.

### APIDs

The DFB uses APIDs in the range of 0x40 to 0x4F. Packets are sent to the DCB with an APID identifier followed by a single data point. Each packet type can have several different data points that must be transferred (e.g. FB, 0x41 has several frequency bands to transfer, E_SVY, 0x43, has three components to transfer). The APID is used to tell what packet type is being sent, and to tell what type of data point is being sent, all data requested for that APID is sent serially in a pre-determined order (see Table 26, Table 27, Table 28, and Table 29).

Table 4. APID Assignments for data produced by the DFB.

| APID | Packet Name | Category | Type |
| --- | --- | --- | --- |
| 0x40 | HSKP | Housekeeping | Housekeeping |
| 0x41 | FB | Survey Data | Filter Bank |
| 0x42 | FB_INT | Burst Trigger | Internal Filter Bank |
| 0x43 | E_SVY | Survey Data | Survey Waveforms |
| 0x44 | V_SVY | Survey Data | Survey Waveforms |
| 0x45 | MAG_SVY | Survey Data | Waveforms, Magnetometer Backup |
| 0x46 | E_B1 | Burst 1 | Burst #1 Waveforms (Low Rate) |
| 0x47 | V_B1 | Burst 1 | Burst #1 Waveforms (Low Rate) |
| 0x48 | SCM_B1 | Burst 1 | Burst #1 Waveforms (Low Rate) |
| 0x49 | E_B2 | Burst 2 | Burst #2 Waveforms (High Rate) |
| 0x4A | V_B2 | Burst 2 | Burst #2 Waveforms (High Rate) |
| 0x4B | SCM_B2 | Burst 2 | Burst #2 Waveforms (High Rate) |
| 0x4C | SVY_INT | Internal Survey Data | Internal Waveforms |
| 0x4D | Deleted |  |  |
| 0x4E | SPEC | Survey Data FFTs | Spectra |
| 0x4F | XSPEC | Survey Data FFTs | Cross Spectra |

### Telemetry Packet Definition

Below are the telemetry packet definitions (bits 23 to 0) for all telemetry produced by the DFB, in the format sent from the DFB FPGA. These do not necessarily represent the format of packets telemetered by the EFW FSW.

#### Waveforms

Table 5. E_SVY APID 0x43 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| E12S | 01000011bin | 16-bit survey data |  |
| E34S | 01000011bin | 16-bit survey data |  |
| E56S | 01000011bin | 16-bit survey data |  |

Table 6. V_SVY APID 0x44 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| V1S | 01000100bin | 16-bit survey data |  |
| V2S | 01000100bin | 16-bit survey data |  |
| V3S | 01000100bin | 16-bit survey data |  |
| V4S | 01000100bin | 16-bit survey data |  |
| V5S | 01000100bin | 16-bit survey data |  |
| V6S | 01000100bin | 16-bit survey data |  |
| VDC_AVG | 01000100bin | 16-bit survey data |  |

Table 7. MAG_SVY APID 0x45 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| MAGU | 01000101bin | 16-bit survey data |  |
| MAGV | 01000101bin | 16-bit survey data |  |
| MAGW | 01000101bin | 16-bit survey data |  |

#### FFTs

The Spectra and Cross-Spectra packet definitions are dependent on the SPEC_BIN parameter defined in Section Spectra Registers which defines the number of bins.

Data provided in bits 15:0 in the FFT telemetry packets implement pseudo-logarithmic compression algorithms. For the Spectra, each 8-bit datum shown in Table 8 is compressed as described in Section 5.3.1.2. For the Cross-spectra, compression depends on whether the data are of the type P, Ic or Rc. and are defined in Section  5.3.1.3

Table 8. SPEC APID 0x4E telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 | SPEC_BIN setting (Number of bins) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 (36) | 1 (64) | 2 (112) |
| SPEC1 | 01001110bin | Bin 1 | Bin 0 | Telemetered: 18 data words | Telemetered: 32 data words | Telemetered: 56 data words |
|  | 01001110bin | Bin 3 | Bin 2 |  |  |  |
|  | 01001110bin | Bin 5 | Bin 4 |  |  |  |
|  | 01001110bin | Bin 7 | Bin 6 |  |  |  |
|  | 01001110bin | Bin 9 | Bin 8 |  |  |  |
|  | 01001110bin | Bin 11 | Bin 10 |  |  |  |
|  | 01001110bin | Bin 13 | Bin 12 |  |  |  |
|  | 01001110bin | Bin 15 | Bin 14 |  |  |  |
|  | 01001110bin | Bin 17 | Bin 16 |  |  |  |
|  | 01001110bin | Bin 19 | Bin 18 |  |  |  |
|  | 01001110bin | Bin 21 | Bin 20 |  |  |  |
|  | 01001110bin | Bin 23 | Bin 22 |  |  |  |
|  | 01001110bin | Bin 25 | Bin 24 |  |  |  |
|  | 01001110bin | Bin 27 | Bin 26 |  |  |  |
|  | 01001110bin | Bin 29 | Bin 28 |  |  |  |
|  | 01001110bin | Bin 31 | Bin 30 |  |  |  |
|  | 01001110bin | Bin 33 | Bin 32 |  |  |  |
|  | 01001110bin | Bin 35 | Bin 34 |  |  |  |
|  | 01001110bin | Bin 37 | Bin 36 | NOT telemetered |  |  |
|  | 01001110bin | Bin 39 | Bin 38 |  |  |  |
|  | 01001110bin | Bin 41 | Bin 40 |  |  |  |
|  | 01001110bin | Bin 43 | Bin 42 |  |  |  |
|  | 01001110bin | Bin 45 | Bin 44 |  |  |  |
|  | 01001110bin | Bin 47 | Bin 46 |  |  |  |
|  | 01001110bin | Bin 49 | Bin 48 |  |  |  |
|  | 01001110bin | Bin 51 | Bin 50 |  |  |  |
|  | 01001110bin | Bin 53 | Bin 52 |  |  |  |
|  | 01001110bin | Bin 55 | Bin 54 |  |  |  |
|  | 01001110bin | Bin 57 | Bin 56 |  |  |  |
|  | 01001110bin | Bin 59 | Bin 58 |  |  |  |
|  | 01001110bin | Bin 61 | Bin 60 |  |  |  |
|  | 01001110bin | Bin 63 | Bin 62 |  |  |  |
|  | 01001110bin | Bin 65 | Bin 64 |  | NOT telemetered |  |
|  | 01001110bin | Bin 67 | Bin 66 |  |  |  |
|  | 01001110bin | Bin 69 | Bin 68 |  |  |  |
|  | 01001110bin | Bin 71 | Bin 70 |  |  |  |
|  | 01001110bin | Bin 73 | Bin 72 |  |  |  |
|  | 01001110bin | Bin 75 | Bin 74 |  |  |  |
|  | 01001110bin | Bin 77 | Bin 76 |  |  |  |
|  | 01001110bin | Bin 79 | Bin 78 |  |  |  |
|  | 01001110bin | Bin 81 | Bin 80 |  |  |  |
|  | 01001110bin | Bin 83 | Bin 82 |  |  |  |
|  | 01001110bin | Bin 85 | Bin 84 |  |  |  |
|  | 01001110bin | Bin 87 | Bin 86 |  |  |  |
|  | 01001110bin | Bin 89 | Bin 88 |  |  |  |
|  | 01001110bin | Bin 91 | Bin 90 |  |  |  |
|  | 01001110bin | Bin 93 | Bin 92 |  |  |  |
|  | 01001110bin | Bin 95 | Bin 94 |  |  |  |
|  | 01001110bin | Bin 97 | Bin 96 |  |  |  |
|  | 01001110bin | Bin 99 | Bin 98 |  |  |  |
|  | 01001110bin | Bin 101 | Bin 100 |  |  |  |
|  | 01001110bin | Bin 103 | Bin 102 |  |  |  |
|  | 01001110bin | Bin 105 | Bin 104 |  |  |  |
|  | 01001110bin | Bin 107 | Bin 106 |  |  |  |
|  | 01001110bin | Bin 109 | Bin 108 |  |  |  |
|  | 01001110bin | Bin 111 | Bin 110 |  |  |  |
| SPEC2 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |
| SPEC3 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |
| SPEC4 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |
| SPEC5 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |
| SPEC6 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |
| SPEC7 | 01001110bin | Same format as SPEC1 |  | 18 data words | 32 data words | 56 data words |

Table 9. XSPEC APID 0x4F telemetry packet definition (Bits 23:0).

| Data Product | 23 … 16 | 15…8 | 7…0 | SPEC_BIN |
| --- | --- | --- | --- | --- |
| XSPEC1 | 01001111bin | Bin 1 PXSPEC_SRC1 | Bin 0 PXSPEC_SRC1 | 0 (36 P1 bins, N=35, 18 words) 1 (64 P1 bins, N=63, 32 words) 2 (112 P1 bins, N=111, 56 words) |
|  | 01001111bin | Bin 3 PXSPEC_SRC1 | Bin 2 PXSPEC_SRC1 |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | Bin N PXSPEC_SRC1 | Bin N-1 PXSPEC_SRC1 |  |
|  | 01001111bin | Bin 1 PXSPEC_SRC2 | Bin 0 PXSPEC_SRC2 | 0 (36 P2 bins, N=35, 18 words) 1 (64 P2 bins, N=63, 32 words) 2 (112 P2 bins, N=111, 56 words) |
|  | 01001111bin | Bin 3 PXSPEC_SRC2 | Bin 2 PXSPEC_SRC2 |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | . | . |  |
|  | 01001111bin | Bin N PXSPEC_SRC2 | Bin N-1 PXSPEC_SRC2 |  |
|  | 01001111bin | Bin 0 Rc |  | 0 (36 Rc, 36 Ic bins, N=35, 72 words) 1 (64 Rc, 64 Ic bins, N=63, 128 words) 2 (112 Rc, 112 Ic bins, N=111, 224 words) |
|  | 01001111bin | Bin 0 Ic |  |  |
|  | 01001111bin | . |  |  |
|  | 01001111bin | . |  |  |
|  | 01001111bin | Bin N Rc |  |  |
|  | 01001111bin | Bin N Ic |  |  |
| XSPEC2 | 01001111bin | Same format as XSPEC1 |  | 0 (108 data words) 1 (192 data words) 2 (336 data words) |
| XSPEC3 | 01001111bin | Same format as XSPEC1 |  |  |
| XSPEC4 | 01001111bin | Same format as XSPEC1 |  |  |

#### Deleted

Table 10. Deleted

#### Filter Bank

The Filter Bank packet definitions are dependent on the FB_FREQBANDS parameter defined in Section 5.2.1.3, which defines the number of bins, or frequency bands.

Each datum in the filter bank packet uses a 19-to-8 bit compression algorithm, described in Section 5.3.1.1.

Table 11. FB APID 0x41 telemetry packet definition (Bits 23:0) for 7 bins: (FB_FREQBANDS=0).

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| FB1_Ave | 01000001bin | Bin 1 (3-6 Hz) | Bin 0 (0.8-1.5 Hz) |
|  | 01000001bin | Bin 3 (50-100 Hz) | Bin 2(12-25 Hz) |
|  | 01000001bin | Bin 5 (800-1.6k Hz) | Bin 4 (200-400 Hz) |
| FB1_Peak / FB1_Ave | 01000001bin | Bin 0 (0.8-1.5 Hz) | Bin 6 (3.2-6.5k Hz) |
|  | 01000001bin | Bin 2(12-25 Hz) | Bin 1 (3-6 Hz) |
|  | 01000001bin | Bin 4 (200-400 Hz) | Bin 3 (50-100 Hz) |
|  | 01000001bin | Bin 6 (3.2-6.5k Hz) | Bin 5 (800-1.6k Hz) |
| FB2_Ave | 01000001bin | Bin 1 (3-6 Hz) | Bin 0 (0.8-1.5 Hz) |
|  | 01000001bin | Bin 3 (50-100 Hz) | Bin 2(12-25 Hz) |
|  | 01000001bin | Bin 5 (800-1.6k Hz) | Bin 4 (200-400 Hz) |
| FB2_Peak / FB2_Ave | 01000001bin | Bin 0 (0.8-1.5 Hz) | Bin 6 (3.2-6.5k Hz) |
|  | 01000001bin | Bin 2(12-25 Hz) | Bin 1 (3-6 Hz) |
|  | 01000001bin | Bin 4 (200-400 Hz) | Bin 3 (50-100 Hz) |
|  | 01000001bin | Bin 6 (3.2-6.5k Hz) | Bin 5 (800-1.6k Hz) |

Table 12. FB APID 0x41 telemetry packet definition (Bits 23:0) for 13 bins: (FB_FREQBANDS=1).

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| FB1_Ave | 01000001bin | Bin 1 (1.5-3Hz) | Bin 0 (0.8-1.5 Hz) |
|  | 01000001bin | Bin 3 (6-12Hz) | Bin 2 (3-6 Hz) |
|  | 01000001bin | Bin 5 (25-50Hz) | Bin 4 (12-25 Hz) |
|  | 01000001bin | Bin 7 (100-200Hz) | Bin 6 (50-100 Hz) |
|  | 01000001bin | Bin 9 (400-800Hz) | Bin 8 (200-400 Hz) |
|  | 01000001bin | Bin 11 (1.6k-3.2kHz) | Bin 10 (800-1.6k Hz) |
| FB1_Peak / FB1_Ave | 01000001bin | Bin 0 (0.8-1.5 Hz) | Bin 12 (3.2-6.5k Hz) |
|  | 01000001bin | Bin 2 (3-6 Hz) | Bin 1 (1.5-3Hz) |
|  | 01000001bin | Bin 4 (12-25 Hz) | Bin 3 (6-12Hz) |
|  | 01000001bin | Bin 6 (50-100 Hz) | Bin 5 (25-50Hz) |
|  | 01000001bin | Bin 8 (200-400 Hz) | Bin 7 (100-200Hz) |
|  | 01000001bin | Bin 10 (800-1.6k Hz) | Bin 9 (400-800Hz) |
|  | 01000001bin | Bin 12 (3.2-6.5k Hz) | Bin 11 (1.6k-3.2kHz) |
| FB2_Ave | 01000001bin | Bin 1 (1.5-3Hz) | Bin 0 (0.8-1.5 Hz) |
|  | 01000001bin | Bin 3 (6-12Hz) | Bin 2 (3-6 Hz) |
|  | 01000001bin | Bin 5 (25-50Hz) | Bin 4 (12-25 Hz) |
|  | 01000001bin | Bin 7 (100-200Hz) | Bin 6 (50-100 Hz) |
|  | 01000001bin | Bin 9 (400-800Hz) | Bin 8 (200-400 Hz) |
|  | 01000001bin | Bin 11 (1.6k-3.2kHz) | Bin 10 (800-1.6k Hz) |
| FB2_Peak / FB2_Ave | 01000001bin | Bin 0 (0.8-1.5 Hz) | Bin 12 (3.2-6.5k Hz) |
|  | 01000001bin | Bin 2 (3-6 Hz) | Bin 1 (1.5-3Hz) |
|  | 01000001bin | Bin 4 (12-25 Hz) | Bin 3 (6-12Hz) |
|  | 01000001bin | Bin 6 (50-100 Hz) | Bin 5 (25-50Hz) |
|  | 01000001bin | Bin 8 (200-400 Hz) | Bin 7 (100-200Hz) |
|  | 01000001bin | Bin 10 (800-1.6k Hz) | Bin 9 (400-800Hz) |
|  | 01000001bin | Bin 12 (3.2-6.5k Hz) | Bin 11 (1.6k-3.2kHz) |

#### Burst 1 (Low rate)

Table 13. E_B1 APID 0x46 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| E12B1 | 01000110bin | 16-bit survey data |  |
| E34B1 | 01000110bin | 16-bit survey data |  |
| E56B1 | 01000110bin | 16-bit survey data |  |

Table 14. V_B1 APID 0x47 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| V1B1 | 01000111bin | 16-bit survey data |  |
| V2B1 | 01000111bin | 16-bit survey data |  |
| V3B1 | 01000111bin | 16-bit survey data |  |
| V4B1 | 01000111bin | 16-bit survey data |  |
| V5B1 | 01000111bin | 16-bit survey data |  |
| V6B1 | 01000111bin | 16-bit survey data |  |
| VDC_AVG | 01000111bin | 16-bit survey data |  |

Table 15. SC_B1 APID 0x48 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| SCMU_B1 | 01001000bin | 16-bit survey data |  |
| SCMV_B1 | 01001000bin | 16-bit survey data |  |
| SCMW_B1 | 01001000bin | 16-bit survey data |  |

#### Burst 2 (Interferometric)

Table 16. E_B2 APID 0x49 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| E12dcB2 | 01001001bin | 16-bit survey data |  |
| E34dcB2 | 01001001bin | 16-bit survey data |  |
| E56dcB2 | 01001001bin | 16-bit survey data |  |
| EdcparB2 | 01001001bin | 16-bit survey data |  |
| EdcprpB2 | 01001001bin | 16-bit survey data |  |
| E12acB2 | 01001001bin | 16-bit survey data |  |
| E34acB2 | 01001001bin | 16-bit survey data |  |
| E56acB2 | 01001001bin | 16-bit survey data |  |
| EacparB2 | 01001001bin | 16-bit survey data |  |
| EacprpB2 | 01001001bin | 16-bit survey data |  |

Table 17. V_B2 APID 0x4A telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| V1B2 | 01001010bin | 16-bit survey data |  |
| V2B2 | 01001010bin | 16-bit survey data |  |
| V3B2 | 01001010bin | 16-bit survey data |  |
| V4B2 | 01001010bin | 16-bit survey data |  |
| V5B2 | 01001010bin | 16-bit survey data |  |
| V6B2 | 01001010bin | 16-bit survey data |  |

Table 18. SC_B2 APID 0x4B telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| SCMU_B2 | 01001000bin | 16-bit survey data |  |
| SCMV_B2 | 01001000bin | 16-bit survey data |  |
| SCMW_B2 | 01001000bin | 16-bit survey data |  |
| SCMpar_B2 | 01001000bin | 16-bit survey data |  |
| SCMprp_B2 | 01001000bin | 16-bit survey data |  |
| SCMprp2_B2 | 01001000bin | 16-bit survey data |  |

#### Internal Waveforms

Table 19. E_INT, V_INT, MAG_INT APID 0x4C telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| V1S | 01001100bin | 16-bit survey data |  |
| V2S | 01001100bin | 16-bit survey data |  |
| V3S | 01001100bin | 16-bit survey data |  |
| V4S | 01001100bin | 16-bit survey data |  |
| V5S | 01001100bin | 16-bit survey data |  |
| V6S | 01001100bin | 16-bit survey data |  |
| E12S | 01001100bin | 16-bit survey data |  |
| MAGU | 01001100bin | 16-bit survey data |  |
| E34S | 01001100bin | 16-bit survey data |  |
| MAGV | 01001100bin | 16-bit survey data |  |
| E56S | 01001100bin | 16-bit survey data |  |
| MAGW | 01001100bin | 16-bit survey data |  |

#### Internal Filter Bank

Table 20. FB_INT APID 0x42 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| FB3_Ave | 01000010bin | Same Format asFB1_Ave |  |
| FB3_Peak | 01000010bin | Same Format as FB1_Peak |  |
| FB4_Ave | 01000010bin | Same Format asFB1_Ave |  |
| FB4_Peak | 01000010bin | Same Format as FB1_Peak |  |

#### Internal Housekeeping

Table 21. HSKP APID 0x40 telemetry packet definition (Bits 23:0)

| Data Product | 23 … 16 | 15…8 | 7…0 |
| --- | --- | --- | --- |
| HSKP | 01000000bin | 00000000 | Register Address Requested |
| HSKP | 01000000bin | Contents of Register |  |

# Analog to Digital Conversion

The twenty-four analog signals from EFW, MAG and SCM are multiplexed prior to the analog to digital conversion. The MUX is divided into two banks. The primary science signals are grouped into one bank and nominally directed to ADC1 and the ancillary science data is grouped in a second bank to be directed to ADC2 (Table 22). Nominally, sampling of signals will be divided between the two ADCs, alternating between the 2 every other signal (V1dc from ADC1, V1ac from ADC2, V2dc from ADC1, and so forth: V1dc, V1ac, V2dc, V2ac, V3dc, V3ac, V4dc, V4ac, V5dc, V5ac, V6dc, V6ac, E12dc, FMU, E34dc, FMV, E56dc, FMW, E12ac, SCU, E34ac, SCV, E56ac, SCW).

One ADC shall be sufficient for attaining minimum science requirements and the instrument shall be capable of operating with a single ADC. The Analog to Digital conversion is handled with two Linear Technology LTC1604 Analog to Digital Converters (ADC). The ADCs shall provide 16-bit analog to digital conversion and shall be sampled at 16k Samples/sec.

Table 22. MUX and ADC usage for DFB analog signals. Note sample order alternates between MUX Bank 1 and 2: V1dc, V1ac, V2dc, V2ac, V3dc, V3ac, V4dc, V4ac, V5dc, V5ac, V6dc, V6ac, E12dc, FMU, E34dc, FMV, E56dc, FMW, E12ac, SCU, E34ac, SCV, E56ac, SCW.

| MUX Bank | ADC | Analog Signal |
| --- | --- | --- |
| 1 | 1 Nominal 2 In case of failure | V1dc |
|  |  | V2dc |
|  |  | V3dc |
|  |  | V4dc |
|  |  | V5dc |
|  |  | V6dc |
|  |  | E12dc |
|  |  | E34dc |
|  |  | E56dc |
|  |  | E12ac |
|  |  | E34ac |
|  |  | E56ac |
| 2 | 2 Nominal 1 In case of failure | V1ac |
|  |  | V2ac |
|  |  | V3ac |
|  |  | V4ac |
|  |  | V5ac |
|  |  | V6ac |
|  |  | FMU (MAGU) |
|  |  | FMV (MAGV) |
|  |  | FMW (MAGW) |
|  |  | SCU (SCMU) |
|  |  | SCV (SCMV) |
|  |  | SCW (SCMW) |

# Analog Housekeeping

No Analog HK is present on the DFB. Note there is a housekeeping product in the internal data products (Table 29) for configuration validity and the correctness of various parameters (table) in the Field Programmable Gate Array (FPGA).

# Digital Signal Processing

The digital signal processing is handled in a Field Programmable Gate Array (FPGA). Static Random Access Memory (SRAM) will be used to support the processing. Details of data products are given in Table 26 through Table 29. In summary, the DFB will:

- Provide filtered, real-time waveform signals (including an average of V1-V4DC) at selectable sample rates per Table 26. These data are provided continually to the DCB, at a commandable sampling rate.

- Provide transformations of the electric field and search coil magnetometer data from spacecraft to magnetic field-aligned coordinates using MAG data (E·B , ExB, SCM·B, SCMxB) per Table 26.

  - E·B: Epar

  - ExB: Eprp

  - SC·B: SCpar

  - SCxB: SCprp (in spin plane)

  - SCxB: SCprp2 (out of spin plane)

- Provide two Filter Banks per Table 26.

- Perform Fast-Fourier Transforms (FFT) per Table 26 to provide spectra of the transformed electric field and search coil magnetometer signals at selectable frequency resolution and bandwidth.

- Perform cross-spectral analysis per Table 26.

DFB will process data from EFW, SCM and MAG and send Survey Data (Table 26) higher-rate Burst 1 data (Table 27) and Burst 2 data (Table 28) and Internal data (Table 29) to the DCB according to the configuration the DCB commands to the DFB. All data products are configurable. For each packet type, there is a single sampling rate for all data, which may be changed upon command. Data products may be individually enabled or disabled (independent of other data in the packet).

## FFT Frequency Band Tables

Table 23. Frequency Table A, 5%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 112 bins are selected.

| Bin | Frequency |  | dF | dF/F |  | Bin | Frequency |  | dF | dF/F |  | Bin | Frequency |  | dF | dF/F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low | High | Hz |  |  |  | Low | High | Hz |  |  |  | Low | High | Hz |  |
| 1 | 0 | 8 | 8 | 200% |  | 39 | 352 | 368 | 16 | 4% |  | 77 | 1792 | 1856 | 64 | 4% |
| 2 | 8 | 16 | 8 | 67% |  | 40 | 368 | 384 | 16 | 4% |  | 78 | 1856 | 1920 | 64 | 3% |
| 3 | 16 | 24 | 8 | 40% |  | 41 | 384 | 400 | 16 | 4% |  | 79 | 1920 | 1984 | 64 | 3% |
| 4 | 24 | 32 | 8 | 29% |  | 42 | 400 | 416 | 16 | 4% |  | 80 | 1984 | 2048 | 64 | 3% |
| 5 | 32 | 40 | 8 | 22% |  | 43 | 416 | 432 | 16 | 4% |  | 81 | 2048 | 2176 | 128 | 6% |
| 6 | 40 | 48 | 8 | 18% |  | 44 | 432 | 448 | 16 | 4% |  | 82 | 2176 | 2304 | 128 | 6% |
| 7 | 48 | 56 | 8 | 15% |  | 45 | 448 | 464 | 16 | 4% |  | 83 | 2304 | 2432 | 128 | 5% |
| 8 | 56 | 64 | 8 | 13% |  | 46 | 464 | 480 | 16 | 3% |  | 84 | 2432 | 2560 | 128 | 5% |
| 9 | 64 | 72 | 8 | 12% |  | 47 | 480 | 496 | 16 | 3% |  | 85 | 2560 | 2688 | 128 | 5% |
| 10 | 72 | 80 | 8 | 11% |  | 48 | 496 | 512 | 16 | 3% |  | 86 | 2688 | 2816 | 128 | 5% |
| 11 | 80 | 88 | 8 | 10% |  | 49 | 512 | 544 | 32 | 6% |  | 87 | 2816 | 2944 | 128 | 4% |
| 12 | 88 | 96 | 8 | 9% |  | 50 | 544 | 576 | 32 | 6% |  | 88 | 2944 | 3072 | 128 | 4% |
| 13 | 96 | 104 | 8 | 8% |  | 51 | 576 | 608 | 32 | 5% |  | 89 | 3072 | 3200 | 128 | 4% |
| 14 | 104 | 112 | 8 | 7% |  | 52 | 608 | 640 | 32 | 5% |  | 90 | 3200 | 3328 | 128 | 4% |
| 15 | 112 | 120 | 8 | 7% |  | 53 | 640 | 672 | 32 | 5% |  | 91 | 3328 | 3456 | 128 | 4% |
| 16 | 120 | 128 | 8 | 6% |  | 54 | 672 | 704 | 32 | 5% |  | 92 | 3456 | 3584 | 128 | 4% |
| 17 | 128 | 136 | 8 | 6% |  | 55 | 704 | 736 | 32 | 4% |  | 93 | 3584 | 3712 | 128 | 4% |
| 18 | 136 | 144 | 8 | 6% |  | 56 | 736 | 768 | 32 | 4% |  | 94 | 3712 | 3840 | 128 | 3% |
| 19 | 144 | 152 | 8 | 5% |  | 57 | 768 | 800 | 32 | 4% |  | 95 | 3840 | 3968 | 128 | 3% |
| 20 | 152 | 160 | 8 | 5% |  | 58 | 800 | 832 | 32 | 4% |  | 96 | 3968 | 4096 | 128 | 3% |
| 21 | 160 | 168 | 8 | 5% |  | 59 | 832 | 864 | 32 | 4% |  | 97 | 4096 | 4352 | 256 | 6% |
| 22 | 168 | 176 | 8 | 5% |  | 60 | 864 | 896 | 32 | 4% |  | 98 | 4352 | 4608 | 256 | 6% |
| 23 | 176 | 184 | 8 | 4% |  | 61 | 896 | 928 | 32 | 4% |  | 99 | 4608 | 4864 | 256 | 5% |
| 24 | 184 | 192 | 8 | 4% |  | 62 | 928 | 960 | 32 | 3% |  | 100 | 4864 | 5120 | 256 | 5% |
| 25 | 192 | 200 | 8 | 4% |  | 63 | 960 | 992 | 32 | 3% |  | 101 | 5120 | 5376 | 256 | 5% |
| 26 | 200 | 208 | 8 | 4% |  | 64 | 992 | 1024 | 32 | 3% |  | 102 | 5376 | 5632 | 256 | 5% |
| 27 | 208 | 216 | 8 | 4% |  | 65 | 1024 | 1088 | 64 | 6% |  | 103 | 5632 | 5888 | 256 | 4% |
| 28 | 216 | 224 | 8 | 4% |  | 66 | 1088 | 1152 | 64 | 6% |  | 104 | 5888 | 6144 | 256 | 4% |
| 29 | 224 | 232 | 8 | 4% |  | 67 | 1152 | 1216 | 64 | 5% |  | 105 | 6144 | 6400 | 256 | 4% |
| 30 | 232 | 240 | 8 | 3% |  | 68 | 1216 | 1280 | 64 | 5% |  | 106 | 6400 | 6656 | 256 | 4% |
| 31 | 240 | 248 | 8 | 3% |  | 69 | 1280 | 1344 | 64 | 5% |  | 107 | 6656 | 6912 | 256 | 4% |
| 32 | 248 | 256 | 8 | 3% |  | 70 | 1344 | 1408 | 64 | 5% |  | 108 | 6912 | 7168 | 256 | 4% |
| 33 | 256 | 272 | 16 | 6% |  | 71 | 1408 | 1472 | 64 | 4% |  | 109 | 7168 | 7424 | 256 | 4% |
| 34 | 272 | 288 | 16 | 6% |  | 72 | 1472 | 1536 | 64 | 4% |  | 110 | 7424 | 7680 | 256 | 3% |
| 35 | 288 | 304 | 16 | 5% |  | 73 | 1536 | 1600 | 64 | 4% |  | 111 | 7680 | 7936 | 256 | 3% |
| 36 | 304 | 320 | 16 | 5% |  | 74 | 1600 | 1664 | 64 | 4% |  | 112 | 7936 | 8192 | 256 | 3% |
| 37 | 320 | 336 | 16 | 5% |  | 75 | 1664 | 1728 | 64 | 4% |  |  |  |  |  |  |
| 38 | 336 | 352 | 16 | 5% |  | 76 | 1728 | 1792 | 64 | 4% |  |  |  |  |  |  |

Table 24. Frequency Table B, 10%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 64 bins are selected.

| Bin | Frequency |  | dF | dF/F |  | Bin | Frequency |  | dF | dF/F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low | High | Hz |  |  |  | Low | High | Hz |  |
| 1 | 0 | 8 | 8 | 200% |  | 39 | 896 | 960 | 64 | 7% |
| 2 | 8 | 16 | 8 | 67% |  | 40 | 960 | 1024 | 64 | 6% |
| 3 | 16 | 24 | 8 | 40% |  | 41 | 1024 | 1152 | 128 | 12% |
| 4 | 24 | 32 | 8 | 29% |  | 42 | 1152 | 1280 | 128 | 11% |
| 5 | 32 | 40 | 8 | 22% |  | 43 | 1280 | 1408 | 128 | 10% |
| 6 | 40 | 48 | 8 | 18% |  | 44 | 1408 | 1536 | 128 | 9% |
| 7 | 48 | 56 | 8 | 15% |  | 45 | 1536 | 1664 | 128 | 8% |
| 8 | 56 | 64 | 8 | 13% |  | 46 | 1664 | 1792 | 128 | 7% |
| 9 | 64 | 72 | 8 | 12% |  | 47 | 1792 | 1920 | 128 | 7% |
| 10 | 72 | 80 | 8 | 11% |  | 48 | 1920 | 2048 | 128 | 6% |
| 11 | 80 | 88 | 8 | 10% |  | 49 | 2048 | 2304 | 256 | 12% |
| 12 | 88 | 96 | 8 | 9% |  | 50 | 2304 | 2560 | 256 | 11% |
| 13 | 96 | 104 | 8 | 8% |  | 51 | 2560 | 2816 | 256 | 10% |
| 14 | 104 | 112 | 8 | 7% |  | 52 | 2816 | 3072 | 256 | 9% |
| 15 | 112 | 120 | 8 | 7% |  | 53 | 3072 | 3328 | 256 | 8% |
| 16 | 120 | 128 | 8 | 6% |  | 54 | 3328 | 3584 | 256 | 7% |
| 17 | 128 | 144 | 16 | 12% |  | 55 | 3584 | 3840 | 256 | 7% |
| 18 | 144 | 160 | 16 | 11% |  | 56 | 3840 | 4096 | 256 | 6% |
| 19 | 160 | 176 | 16 | 10% |  | 57 | 4096 | 4608 | 512 | 12% |
| 20 | 176 | 192 | 16 | 9% |  | 58 | 4608 | 5120 | 512 | 11% |
| 21 | 192 | 208 | 16 | 8% |  | 59 | 5120 | 5632 | 512 | 10% |
| 22 | 208 | 224 | 16 | 7% |  | 60 | 5632 | 6144 | 512 | 9% |
| 23 | 224 | 240 | 16 | 7% |  | 61 | 6144 | 6656 | 512 | 8% |
| 24 | 240 | 256 | 16 | 6% |  | 62 | 6656 | 7168 | 512 | 7% |
| 25 | 256 | 288 | 32 | 12% |  | 63 | 7168 | 7680 | 512 | 7% |
| 26 | 288 | 320 | 32 | 11% |  | 64 | 7680 | 8192 | 512 | 6% |
| 27 | 320 | 352 | 32 | 10% |  |  |  |  |  |  |
| 28 | 352 | 384 | 32 | 9% |  |  |  |  |  |  |
| 29 | 384 | 416 | 32 | 8% |  |  |  |  |  |  |
| 30 | 416 | 448 | 32 | 7% |  |  |  |  |  |  |
| 31 | 448 | 480 | 32 | 7% |  |  |  |  |  |  |
| 32 | 480 | 512 | 32 | 6% |  |  |  |  |  |  |
| 33 | 512 | 576 | 64 | 12% |  |  |  |  |  |  |
| 34 | 576 | 640 | 64 | 11% |  |  |  |  |  |  |
| 35 | 640 | 704 | 64 | 10% |  |  |  |  |  |  |
| 36 | 704 | 768 | 64 | 9% |  |  |  |  |  |  |
| 37 | 768 | 832 | 64 | 8% |  |  |  |  |  |  |
| 38 | 832 | 896 | 64 | 7% |  |  |  |  |  |  |

Table 25. Frequency Table C, 20%. Frequency given in Hertz. Used for FFT data products SPEC and XSPEC when 36 bins are selected.

| Bin | Frequency |  | dF | dF/F |
| --- | --- | --- | --- | --- |
|  | Low | High | Hz |  |
| 1 | 0 | 8 | 8 | 200% |
| 2 | 8 | 16 | 8 | 67% |
| 3 | 16 | 24 | 8 | 40% |
| 4 | 24 | 32 | 8 | 29% |
| 5 | 32 | 40 | 8 | 22% |
| 6 | 40 | 48 | 8 | 18% |
| 7 | 48 | 56 | 8 | 15% |
| 8 | 56 | 64 | 8 | 13% |
| 9 | 64 | 80 | 16 | 22% |
| 10 | 80 | 96 | 16 | 18% |
| 11 | 96 | 112 | 16 | 15% |
| 12 | 112 | 128 | 16 | 13% |
| 13 | 128 | 160 | 32 | 22% |
| 14 | 160 | 192 | 32 | 18% |
| 15 | 192 | 224 | 32 | 15% |
| 16 | 224 | 256 | 32 | 13% |
| 17 | 256 | 320 | 64 | 22% |
| 18 | 320 | 384 | 64 | 18% |
| 19 | 384 | 448 | 64 | 15% |
| 20 | 448 | 512 | 64 | 13% |
| 21 | 512 | 640 | 128 | 22% |
| 22 | 640 | 768 | 128 | 18% |
| 23 | 768 | 896 | 128 | 15% |
| 24 | 896 | 1024 | 128 | 13% |
| 25 | 1024 | 1280 | 256 | 22% |
| 26 | 1280 | 1536 | 256 | 18% |
| 27 | 1536 | 1792 | 256 | 15% |
| 28 | 1792 | 2048 | 256 | 13% |
| 29 | 2048 | 2560 | 512 | 22% |
| 30 | 2560 | 3072 | 512 | 18% |
| 31 | 3072 | 3584 | 512 | 15% |
| 32 | 3584 | 4096 | 512 | 13% |
| 33 | 4096 | 5120 | 1024 | 22% |
| 34 | 5120 | 6144 | 1024 | 18% |
| 35 | 6144 | 7168 | 1024 | 15% |
| 36 | 7168 | 8192 | 1024 | 13% |

Table 26. Survey data products.

| Packet | APID | Data Name |  |  |  | Nominal Data Selection | No. Comp. Nominal | No. Comp. Avail. | Source Signals | Meas. Range | Meas. Res. | Frequency Bands |  |  |  |  |  | Sample/s (Min) | Sample/s (Max) | Sample/s (Nominal) | Comment | Bits/ Sample | Freq Bins Nominal | Freq Bins Avail | Bits/s to IDPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveforms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E_SVY | 0x43 | In order of delivery: E12S, E34S, E56S |  |  |  | All | 3 | 3 | E12dc, E34dc, E56dc | +/-1 V/m | 30 uV/m |  |  |  |  |  |  | 1 | DFB: 16K DCB: 128 | 32 | DFB can generate 16KS/s, FSW can receive only 128 S/s | 16 |  |  | 1536 |
| V_SVY | 0x44 | In order of delivery: V1S, V2S, V3S, V4S, V5S, V6S |  |  |  | All | 6 | 6 | V1dc,V2dc,V3dc,V4dc, V5dc, V6dc | +/-225V | 6 mV |  |  |  |  |  |  | 1 | DFB: 16K DCB: 128 | 32 |  | 16 |  |  | 3072 |
| MAG_SVY | 0x45 | In order of delivery: MAGU, MAGV, MAGW |  |  |  | All | 3 | 3 | MAGU, MAGV, MAGW | +/- 5 V |  |  |  |  |  |  |  | 1 | DFB: 16K DCB: 128 | 32 |  | 16 |  |  | 1536 |
| FFTs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPEC5 | 0x4E | In order of delivery: SPEC1, SPEC2, SPEC3, SPEC4, SPEC5, SPEC6, SPEC7 |  |  |  | Nominal: E12ac, E56ac, SCMpar, SCMprp. SCMW, V1ac, V2ac | 7 | 7 | Select 8 of: E12dc, E34dc, E56dc, E12ac, E34ac, E56ac, Edcpar, Edcprp, Eacpar, Eacprp, V1ac, V2ac, V3ac, V4ac, V5ac, V6ac SCMU, SCMV, SCMW, SCMpar, SCMprp, (V1dc+V2dc+V3dc+V4dc)/4, SCMprp2 | 100 dB Depends on source signal, see waveform table. | Depends on source signal, see waveform table. | A (5%) See Table 23 112 bins |  | B (10%) See Table 24 64 bins |  |  | C (20%) See Table 25 36 bins | 1/128 | 8 | 1/8 | See 5.3.1 for compression detail | 8 | 64 | 112, 64, 32 | 448 |
| XSPEC1,5 | 0x4F | In order of delivery: XSPEC1 (Rc,Ic, Px, Py), XSPEC2, XSPEC3, XSPEC4 |  |  |  | SCMWxE12ac, V1acxV2ac | 2 | 4 | Select pairs from SPEC1 - SPEC7. Select: number of FFTs per XSPEC in addition to cadence. | Phase difference averaged over time / frequency bins as per tables. | Variation in phase. | A (5%) Same as Selection for FFT See Table 23 112 bins |  | B (10%) Same as Selection for FFT See Table 24 64 bins |  |  | C (20%) Same as Selection for FFT See Table 25 36 bins | 1/128 | 8 | 1/8 | 48 bits: 16/16/8x2: real & imaginary, coherency See 5.3.1 for compression detail | 48 | 64 | 112, 64, 32 | 768 |
| Solitary Wave Counter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Deleted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter Bank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FB | 0x41 | In order of delivery: FB1_Ave and FB1_Peak, FB2_Ave and FB2_Peak *Components are selected in pairs; Select either FB1 or FB2 |  |  |  | E12dc | 2 | 4 | Select 2 of: E12dc, E12ac, E34dc, E34ac, E56dc, E56ac, SCMU, SCMV, SCMW, (V1dc+V2dc+V3dc+V4dc)/4 |  |  | A, 13 bins 0.8-1.5, 1.5-3, 3-6, 6-12, 12-25, 25-50, 50-100, 100-200, 200-400, 400-800, 800-1.6k, 1.6k-3.2k, 3.2-6.5k |  |  | B, 7 bins (Nominal) 0.8-1.5, 3-6, 12-25, 50-100, 200-400, 800-1.6k, 3.2-6.5k |  |  | 1/16 | 64 | 8 | See 5.3.1 for compression detail | 8 | 7 | 7 | 896 |
| Total, Survey Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8256 |

Notes:

1 XSPEC Nominal includes SPEC which are not nominally telemetered to ground as SPEC values.

2 SCMpar and SCMprp are rotated quantities, using SCM·B, SCMxB

3 Edcpar, Edcprp and Eacpar, and Eacprp, are rotated quantities using E·B , ExB.

4 Peak is highest value in measurement period. Components are selected in pairs; Select either FB1 or FB2, single selection provides 2 components: Ave & Peak

5 In the non-nominal configuration [ADC2_SRC_SEL = 0 and ADC2_SRC_SEL = 1], spectra and cross spectra will not function properly if any VAC product is selected (data will be noise on all > spectra).

Table 27. Burst 1 (Low Rate) data products.

| Packet | APID |  | Data Name | Nominal Data Selection | No. Comp. Nominal | No. Comp. Avail. | Source Signals | Meas. Range | Meas. Res. | Sample/s |  |  | Bits/Sample | Bits/s to IDPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Min | Max | Nominal |  |  |
| E_B1 | 0x46 |  | In order of delivery: E12B1, E34B1, E56B1 | All | 3 | 3 | E12dc, E34dc, E56dc | +/-1 V/m | 30 uV/m | 1 | 16K | 512 | 16 | 24,576 |
| V_B1 | 0x47 |  | In order of delivery: V1B1, V2B1, V3B1, V4B1, V5B1, V6B1 | All | 6 | 6 | V1dc,V2dc,V3dc,V4dc,V5dc,V6dc | +/-225V | 6 mV | 1 | 16K | 512 | 16 | 49,152 |
| SCM_B1 | 0x48 |  | In order of delivery: SCMU_B1, SCMV_B1, SCMW_B1 | All | 3 | 3 | SCMU, SCMV, SCMW | +/-5V | NA | 1 | 16K | 512 | 16 | 24,576 |
| Total, Burst 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 98,304 |

Table 28. Burst 2 (Interferometric) data products.

| Packet | APID | Data Name | Nominal Data Selection | No. Comp. Nominal | No. Comp. Avail. | Source Signals |  |  |  | Meas. Range |  | Meas. Res. |  | Sample/s |  |  | Bits/Sample | Bits/s to IDPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Min | Max | Nominal |  |  |
| E_B21 | 0x49 | E12dcB2, E34dcB2, E56dcB2, E12acB2, E34acB2, E56acB2, EdcparB2, EdcprpB2, EacparB2, EacprpB2, | E12AC, E34AC, E56AC | 3 | 5 | E12dc, E34dc, E56dc, E12ac, E34ac, E56ac, Edcpar3, Edcprp3, Eacpar3, Eacprp3 |  |  |  | AC: +/- 400 mV/m. DC: +/-1 V/m |  | AC: 12 uV/m. DC: 30 uV/m. |  | 1 | 16K | 16384 | 16 | 786,432 |
| V_B2 | 0x4A | In order of delivery: V1B1, V2B1, V3B1, V4B1, V5B1, V6B1 | All | 6 | 6 | V1ac, V2ac, V3ac, V4ac, V5ac, V6ac |  |  |  | +/-12.5V |  | 0.4mV |  | 1 | 16K | 16384 | 16 | 1,572,864 |
| SCM_B2 | 0x4B | SCMU_B2, SCMV_B2, SCMW_B2, SCpar_B2, SCprp_B2 | SCMU, SCMV, SCMW | 3 | 5 | SCMU, SCMV, SCMW, SCMpar2, SCMprp2 |  |  |  | +/- 5V |  | NA |  | 1 | 16K | 16384 | 16 | 786432 |
| Total, Burst 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3,145,728 |

1 Either 3 or 5 products, and either DC or AC. (E12, E34, E56)  or  (E12, E34, E56) and (Epar, Eprp)

2 SCMpar and SCMprp are rotated quantities, using SCM·B, SCMxB

3 Edcpar, Edcprp and Eacpar, and Eacprp, are rotated quantities using E·B , ExB.

Table 29. Internal data products.

| Packet | APID | Data Name |  |  |  |  | Nominal Data Selection | No. Comp. Nominal | No. Comp. Avail. | Source Signals | Meas. Range | Meas. Resolution | Frequency Bands |  | Sample/s (Nominal) | Bits/Sample | Bits/s to IDPU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INTERNAL Waveforms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SVY_INT: (E_INT, V_INT, MAG_INT) | 0x4C | Nominal order of delivery (ADC1_SRC_SEL=0, ADC2_SRC_SEL=1): V1S, V2S, V3S, V4S, V5S, V6S, E12S, MAGU, E34S, MAGV, E56S, MAGW Alternate order of delivery (ADC1_SRC_SEL=1, ADC2_SRC_SEL=0): V1S, V2S, V3S, V4S, V5S, V6S, MAGU, E12S, MAGV, E34S, MAGW, E56S |  |  |  |  | All | 12 | 12 | Nominal order read from MUX (ADC1_SRC_SEL=0, ADC2_SRC_SEL=1): V1dc, V2dc, V3dc, V4dc, V5dc, V6dc, E12dc, MAGU, E34dc, MAGV, E56dc, MAGW Alternate order (ADC1_SRC_SEL=1, ADC2_SRC_SEL=0): V1dc, V2dc, V3dc, V4dc, V5dc, V6dc, MAGU, E12dc, MAGV, E34dc, MAGW, E56dc | E: +/-1 V/m V: +/-225V | E: 30 uV/m V: 6 mV |  |  | 32 | 16 | 6144 |
| INTERNAL Filter Bank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FB_INT | 0x42 | In order of delivery: FB3_Ave and FB3_Peak, FB4_Ave and FB4_Peak |  |  |  |  | E12dc | 2 | 4 | Select 1 of: E12dc, E12ac, E34dc, E34ac, E56dc, E56ac, SCMU, SCMV, SCMW | Depends on source signal, see waveform table. |  | A(Nominal) 0.8-1.5, 1.5-3, 3-6, 6-12, 12-25, 25-50, 50-100, 100-200, 200-400, 400-800, 800-1.6k, 1.6k-3.2k, 3.2-6.5k | B 0.8-1.5, 3-6, 12-25, 50-100, 200-400, 800-1.6k, 3.2-6.5k | 32 | 8 | 6656 |
| INTERNAL Housekeeping |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HSKP | 0x40 | In order of delivery: Address requested, Contents of register |  |  |  |  | Data is generated only is response to CMD ID 0x00. Two packets are generated, the REGISTER_READ address specified in command and the contents of that address. |  |  | FPGA address written to REGISTER_READ (Cmd ID 0x00) |  |  |  |  | 0 | 16 | 0 |
| Total, INTERNAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12800 |

# Connectors

## Detector Interface

The DFB will interface with the EMFISIS SCM and MAG via a 26 pin High Density D-subminiature connector on the front panel of the board. The pin assignments will be as shown in Table 30.

Table 30. Interface with EMFISIS SCM and MAG, EFW-J703 pinout.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | SCMU |  | Analog Voltage |  | I |  | SCM |  | (+) |
| 2 |  | SCMU Return |  | Signal Return |  | I |  | SCM |  | (-) Coax shield on SCMU signal |
| 3 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 4 |  | SCMV |  | Analog Voltage |  | I |  | SCM |  | (+) |
| 5 |  | SCMV Return |  | Signal Return |  | I |  | SCM |  | (-) Coax shield on SCMV signal |
| 6 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 7 |  | SCMW |  | Analog Voltage |  | I |  | SCM |  | (+) |
| 8 |  | SCMW Return |  | Signal Return |  | I |  | SCM |  | (-) Coax shield on SCMW signal |
| 9 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 10 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 11 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 12 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 13 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 14 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 15 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 16 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 17 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 18 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 19 |  | MAGU |  | Analog Voltage |  | I |  | MAG |  | (+) |
| 20 |  | MAGU Return |  | Signal Return |  | I |  | MAG |  | (-) Coax shield on MAGU signal |
| 21 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 22 |  | MAGV |  | Analog Voltage |  | I |  | MAG |  | (+) |
| 23 |  | MAGV Return |  | Signal Return |  | I |  | MAG |  | (-) Coax shield on MAGV signal |
| 24 |  | AGND |  |  |  |  |  |  |  | Analog ground |
| 25 |  | MAGW |  | Analog Voltage |  | I |  | MAG |  | (+) |
| 26 |  | MAGW Return |  | Signal Return |  | I |  | MAG |  | (-) Coax shield on MAGW signal |

Table 31. Interface with EFW-J715.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE1 |  | EF signal |  | I |  | BEB |  | EF1 |
| Shell |  | Shield |  | Shield |  | I |  | BEB |  | Shield tied to AGND on DFB end |

Table 32. Interface with EFW-J716.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE2 |  | EF signal |  | I |  | BEB |  | EF2 |
| Shell |  | Shield |  | Shield |  | I |  | BEB |  | Shield tied to AGND on DFB 04. end |

Table 33. Interface with EFW-J717.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE3 |  | EF signal |  | I |  | BEB |  | EF3 |
| Shell |  | Shield |  | Shield |  | I |  | BEB |  | Shield tied to AGND on DFB end |

Table 34. Interface with EFW-J718.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE4 |  | EF signal |  | I | BEB | EF4 |
| Shell |  | Shield |  | Shield |  | I | BEB | Shield tied to AGND on DFB end |

Table 35. Interface with EFW-J719.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE5 |  | EF signal |  | I |  | BEB |  | EF5 |
| Shell |  | Shield |  | Shield |  | I |  | BEB |  | Shield tied to AGND on DFB end |

Table 36. Interface with EFW-J720.

| Pin # | Signal Name |  | Signal Function |  | I/O |  | From/To |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center Conductor |  | VSPHERE6 |  | EF signal |  | I |  | BEB |  | EF6 |
| Shell |  | Shield |  | Shield |  | I |  | BEB |  | Shield tied to AGND on DFB end |

## Backplane Interface

The DFB will interface with the IDPU for power, command, and telemetry via the backplane. The pin assignments will be as follows:

| BKP Pin # | Signal Name | Signal Function | S/R | From/To | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | DFB_CMD | DFBCommand | R | DCB | Same signal as Pin 4 |
| 2 | DFB_CLK | DFB Clock | R | DCB | 8.389MHz Clock Same signal as Pin 7 |
| 3 | DFB_TLM0 | DFB Telemetry Line0 | S | DCB | Parallel output with DFB_TLM1 |
| 4 | DFB_CMD | DFB Command | R | DCB | Same signal as Pin 1 |
| 5 | DFB_1HZ | DFB 1Hz | R | DCB | 1 pps signal Same signal as Pin 8 |
| 6 | DFB_TLM1 | DFB Telemetry Line1 | S | DCB | Parallel output with DFB_TLM0 |
| 7 | DFB_CLK | DFB Clock | R | DCB | 8.389MHz Clock Same signal as Pin 2 |
| 8 | DFB_1HZ | DFB 1Hz | R | DCB | 1 pps signal Same signal as Pin 5 |
| 31 | IDPU_P5VD | DCB & DFB +5VD | R | LVPS | Same signal as Pin 34 |
| 34 | IDPU_P5VD | DCB & DFB +5VD | R | LVPS | Same signal as Pin 31 |
| 35 | IDPU_P3.6VD | DCB & DFB +3.6VD | R | LVPS | Same signal as Pin 41 |
| 36 | IDPU_P1.8VD | DCB & DFB +1.8VD | R | LVPS | Same signal as Pin 42 |
| 37 | IDPU_DGND | DCB & DFB Digital Ground | R | LVPS | Same signal as Pins 38, 39 |
| 38 | IDPU_DGND | DCB & DFB Digital Ground | R | LVPS | Same signal as Pins 37, 39 |
| 39 | IDPU_DGND | DCB & DFB Digital Ground | R | LVPS | Same signal as Pins 37, 38 |
| 40 | IDPU_P5VA | IDPU_P5VA | R | LVPS | Same signal as Pin 52 |
| 41 | IDPU_P3.6VD | DCB & DFB +3.6VD | R | LVPS | Same signal as Pin 35 |
| 42 | IDPU_P1.8VD | DCB & DFB +1.8VD | R | LVPS | Same signal as Pin 36 |
| 45 | IDPU_AGND | DCB & DFB Analog Ground | R | LVPS | Same signal as Pin 48 |
| 46 | IDPU_P10VA | IDPU +10VA | R | LVPS | Same signal as Pin 49 |
| 47 | IDPU_N10VA | IDPU -10VA | R | LVPS | Same signal as Pin 50 |
| 48 | IDPU_AGND | DCB & DFB Analog Ground | R | LVPS | Same signal as Pin 45 |
| 49 | IDPU_P10VA | DCB & DFB +10VA | R | LVPS | Same signal as Pin 46 |
| 50 | IDPU_N10VA | DCB & DFB -10VA | R | LVPS | Same signal as Pin 47 |
| 52 | IDPU_P5VA | DCB & DFB +5VA | R | LVPS | Same signal as Pin 40 |
| 53 | IDPU_N5VA | DCB & DFB -5VA | R | LVPS | Same signal as Pin 56 |
| 56 | IDPU_N5VA | IDPU -5VA | R | LVPS | Same signal as Pin 53 |

# DFB FPGA System Timing

This section describes the following:

- How system timing information generated by the DCB is used to synchronize the operation of the DFB FPGA,

- DFB FPGA internally derived timing information and its use,

- The data sample and data product delays introduced by the DFB FPGA’s internal modules.

## Overview

All the system timing information received by the DFB FPGA is generated by the DCB.  This includes a 223Hz clock, which the DFB FPGA uses as its core clock, as well as information (PPS and Super-PPS) to make sure the operation of the DCB and the DFB remains synchronized.  The synchronization information provided by the DCB is a 1 second tick (PPS) as well as an N second tick (Super-PPS) where N is an integer whose value is under the control  of the software running on the DCB.

The figure below is a logical representation of how the system timing is coupled with the DFB FPGA’s internal modules.

Figure 6 DFB Timing Overview

### Timing Info from DCB

The following timing information is generated by the DCB and consumed by the DFB:

- 223Hz clock: DFB FPGA’s core clock

- PPS: 1 second tick

- Super-PPS: N second tick, where the value of N is an integer specified by the DCB.

#### PPS

The 1 second tick is sampled by the DFB FPGA when the PPS signal on the back-plane interface is high for one 223Hz clock cycle.

#### Super-PPS

The N second tick is event that that is initiated by the DCB performing a write to the DFB FPGA register located at address 0x3F.  The write operation of this register is an indication to the DFB FPGA that the next 1 second tick sampled is an N second tick (aka Super-PPS event) as well as a 1 second tick.

### Derived Timing Info

The DFB FPGA uses the timing information from the DCB to generate the following derived timing information:

- 16KHz Tick

- 1/128th Second bank Switch

#### 16KHz Tick

The 223Hz clock and the PPS are used to derive a 16KHz Tick that is synchronous to the PPS.  As per the timing diagram below, the 16KHz Tick is generated in the same clock cycle as the PPS.

Note: The FPGA’s Timing Gen logic will assure that the 16KHz tick is synchronous to the PPS.  Hence, a skew between the PPS and the 16KHz Tick caused by a SEU will be rectified on the next 1 second tick.  There is no skew between the PPS and the 16KHz Tick during normal operation.

The 16KHz Tick is used by the DFB FPGA to fetch 24 data samples from the ADC.  The start of this 16KHz window occurs when the 16KHz tick is sampled high.

#### 1/128th Second Bank Switch

The 223Hz clock and the PPS are used to derive the Bank Switch that is synchronous to the PPS.  As per the timing diagram below, the polarity of the Bank Switch changes every 1/128th of a second.  The rising edge of a PPS, under normal operation, coincides with the rising edge of the Bank Switch.

Note: The FPGA’s Timing Gen logic will assure that the rising edge of the PPS and the rising edge of the Bank Switch always coincides.  If a skew between the signals has been introduced, the skew is rectified by the Timing Gen Logic on the next 1 second tick.

The Bank Switch is utilized by the DFB FPGA as follows:

- Embedded with the data as it is read by the ADI to propagate timing information to the downstream modules.

- Determines the operations of the two FIFOs in the Data Processor module (i.e. which FIFO is used to store data products generated and which FIFO is used to forward the previously stored data products to the DCB).

### Data and Embedded Timing Info

The value of the PPS and the Bank Switch signals are encoded and embedded with the data stream in the ADI module.  The downstream modules use this embedded information as follows:

- Used to synchronize the state machines with the PPS: WAVE, TRIG, and SPEC

- Used to calculate the reporting rate of the data products: TRIG and SPEC

### Data Only

Data stream does not contain any embedded timing information such as PPS or Bank Switch.

## Module Synchronization & Data Path Delay

This section describes how the modules in the DFB FPGA use the timing information to stay synchronized to the DCB.  Furthermore, the data path delays through each individual module are quantified.

### Analog-to-Digital Interface (ADI) Module

The ADI module is enabled following a reset.  That is, if clock is present the ADI module will fetch data samples on 24 sources at a rate of 16KHz from the ADCs and forward them onto the Digital Interface.  Moreover, the FAP and the VDC Average are also enabled following a reset.  These modules snoop the 24 data streams fetched from the ADCs, generate derived data streams at a 16KHz rate and forward them to the Digital Filter.  In all, 32 16KHz data streams are transmitted to the Digital Filters module.

Each data stream is embedded with the current value of PPS and Bank Switch.  The embedded timing information is used by the modules downstream to remain synchronized to the DCB.

#### Timing & Synchronization

For its operation, the Waveform module makes use of the timing information from the Timing Gen module (DT 0) and the data read from the ADCs.

##### PPS

The ADI module make use of the PPS signal received from the Timing Gen module as follows:

- The state machines in the ADI module transition to their default state such that the operation of the DFB FPGA remains synchronized with the DCB.

- The value of PPS is embedded into the data streams generated and used by the downstream modules.

##### Bank Switch

The ADI module embeds the value of Bank Switch into the data streams generated.  The embedded timing information is used by the downstream modules to stay synchronized with the DCB.

##### 16KHz Tick

The ADI is specified to fetch data samples from the ADCs for up to 24 sources at a rate of 16KHz.  Hence, the 16KHz Tick indicates to the ADI to start fetching data samples from the ADCs.  Once the ADI has sampled the 16KHz Tick as high, the state machine will fetch a data sample every 32 clock cycles of the 223Hz clock for up to 24 sources.

#### Data Path Delay

The ADI is able to fetch or generate a sample from all the sources and transmit them to the Digital Filter within a 16KHz time period.  The data path delay through the ADI module, for all data streams with the exception of the ones generated by the FAP, is less than 60us and hence negligible when calculating the overall data path delay through the FPGA.

The processing time required by the FAP does not allow these data streams to be transmitted to the Digital Filter module in the same 16KHz time period as the ADC data streams used as sources.  Due to this processing time, the FAP generated data streams are transmitted to the Digital Filter one 16KHz time period later in relation to the ADC data streams.  Hence, the data path delay for the FAP generated data streams are on the order of 100us, which is also negligible when calculating the overall data path delay through the DFB FPGA.

### Digital Filters (FIL) Module

The Digital Filters module is always enabled.  As soon as the 223Hz clock is present the Digital Filter module will accept data sample upsis disabled following a reset and hence in this default state no data products are generated.  The Waveform will not operate and generate data products until the module has had its registers configured as well as continuously receiving data samples and timing information.  The data products are generated with the embedded Bank Switch timing information.

#### Timing & Synchronization

For its operation, the Digital Filter module makes use of the data & embedded timing information (ET 0) received from the ADI module.

##### Embedded Timing Info

The Digital Filters module makes use of the embedded timing information (ET 0) as follows:

- When the embedded PPS sampled high, the state machines in the Digital Filters module transition to their default state such that the operation of the DFB FPGA remains synchronized with the DCB.

- The embedded timing information (i.e. PPS and Bank Switch) received from the upstream modules (ET 0) is propagated to the output port (ET 1).

#### Data Path Delay

The Digital Filters module introduces a data path delay described by the following equation where P is the number of passes through the filter:

α=1P3215−α

### Waveform Player (WAVE) Module

The Waveform module is disabled following a reset and hence in this default state no data products are generated.  The Waveform will not operate and generate data products until the module has had its registers configured as well as continuously receiving data samples and timing information.  The data products are generated with the embedded Bank Switch timing information.

#### Timing & Synchronization

For its operation, the Waveform module makes use of the data & embedded timing information (ET 1) and timing information from the Timing Gen module (DT 1).

##### PPS

The PPS signal received directly from the Timing Gen module controls when the module’s registers are configured.  The configuration of the module’s registers can be performed at anytime, however they will only take effect then the PPS signal is sampled high (i.e. on the PPS event).

##### Embedded Timing Info

The module makes use of the embedded Bank Switch timing information by propagating and embedding it with the data products transmitted to the Data Processor (ET 2).

#### Data Path Delay

The Waveform module generates data products at reporting rates configured by the host.  The data path delay introduced by the Waveform module is on the order of 100us and hence negligible when calculating the overall data path delay through the DFB FPGA.

A delay on the order of 100us is accomplished by the following facts:

- Two data samples, and accompanied timing, are received and must be processed every 16KHz

- Processing performed on data samples consists of a look-up to determine if data sample forwarded

- Clock frequency of logic in module: 223Hz

### Triggers (TRIG) Module

The Triggers module is disabled following a reset and hence in this default state no data products are generated.  The Triggers will not operate and generate data products until the module has had its registers configured as well as continuously receiving data samples and timing information.  The data products are generated with no embedded timing.

#### Timing & Synchronization

For its operation, the Triggers module makes use of the data & embedded timing information (ET 1) and timing information from the Timing Gen module (DT 1).

##### PPS

The PPS signal received directly from the Timing Gen module controls when the module’s registers are configured.  The configuration of the module’s registers can be performed at anytime, however they will only take effect then the PPS signal is sampled high (i.e. on the PPS event).

##### Embedded Timing Info

The module makes use of the embedded PPS and embedded Bank Switch timing information received from the Digital Filters as follows.

- PPS & Bank Switch are used to calculate when data products are to be transmitted to the Data Processor

- PPS is used conjunction with the Super-PPS to synchronize the operation of the entire module.

##### Super-PPS

When the Super-PPS signal has been sampled high by the module, the next embedded PPS received constitutes a Super-PPS event.  The Triggers module uses the Super-PPS event to synchronize its operation.

#### Data Path Delay

The Triggers module generates data products at reporting rates configured by the host.  The data path delay introduced by the Triggers module in relation to the associated low-pass data samples transmitted out on the Waveform player is on the order of 100us and hence negligible when calculating the overall data path delay through the DFB FPGA.

A delay on the order of 100us is accomplished by the following facts:

- Two data samples, and accompanied timing, are received and must be processed every 16KHz

- Processing performed on data samples consists of simple arithmetic as well as reading and writing to an internal RAM

- Clock frequency of logic in module: 223Hz

### Spectra/X-Spectra (SPEC) Module

The Spectra/X-Spectra module is disabled following a reset and hence in this default state no data products are generated.  The Spectra/X-Spectra will not operate and generate data products until the module has had its registers configured as well as continuously receiving data samples and timing information.  The data products are generated with no embedded timing.

#### Timing & Synchronization

For its operation, the Spectra/X-Spectra module makes use of the data & embedded timing information (ET 1), timing information from the Timing Gen module (DT 1), as well as a 1/8th of a second time tick generated from the 16KHz data samples received by the Ingress SM.

##### PPS

The PPS signal received directly from the Timing Gen module controls when the module’s registers are configured.  The configuration of the module’s registers can be performed at anytime, however they will only take effect then the PPS signal is sampled high (i.e. on the PPS event).

##### Embedded Timing Info

The module makes use of the embedded PPS timing information received from the Digital Filters module in the following two ways:

- Synchronize the FFT delineation in the Ingress SM

- In conjunction with the Super-PPS, synchronize the operation of the SPEC module.

##### Super-PPS

When the Super-PPS signal has been sampled high by the module, the next embedded PPS received constitutes a Super-PPS event.  The Spectra/X-Spectra module uses the Super-PPS event to synchronize its operation.

##### 1/8th of a Second Time Tick

The Ingress SM generates the 1/8th of a second time tick as this is the time required to receive a set of 2,048 16KHz data samples.  In order to be able to perform an FFT on the set of 2,048 16KHz data samples at wire speed, 1/8th of a second is the time allowed to perform all the calculations to generate the Spectra and X-Spectra data products.

As the Ingress SM generates the 1/8th of second time tick, this information is conveyed from the Ingress SM to the Control SM via the FFT Enable signal.  The Control SM uses the 1/8th of a second time tick to increment its internal timer.  This internal timer is used to compare to the NCAD and NAVG parameters; determining if FFT is to be performed on the next set of data samples as well as determining if the data products are available to be transferred to the Data Processor.  The Control SM asserts the XFER Enable signal to notify the Egress SM to being transferring the data products to the Data Processor.

##### Bank Switch

The Egress SM makes use of the Bank Switch signal to delay the transfer of the data products to the Data Processor by 1/128th of a second.  Delaying the transfer of the data products by 1/128th of a second enables the Control SM to fetch the data samples out of the SRAM to begin the FFT calculation on the next data product.  This then allows the Egress SM to fetch the data products out of the SRAM while the Control SM does not need to access the SRAM; hence, resulting in more efficient utilization of the SRAM bandwidth.

#### Data Path Delay

The total data path delay through the Spectra/X-Spectra module in relations to the same data stream transmitted through the Waveform Player is 0.2578 seconds.  The breakdown of the data path delay per state machine is as follows:

- Ingress SM Delay: 0.1250 seconds

- Control SM Delay: 0.1250 seconds

- Egress SM Delay: 0.0078 seconds

Note: The NCAD and NAVG parameters must be taken into account when correlating the data products generated by the Spectra/X-Spectra module vs the same input data stream transmitted out via the Waveform Player.  See “DFB FPGA Features & Anomalies” document for how the Spectra and X-Spectra data products are generated when NAVGSpec > NAVGXSpec.

### Command Processor

The Command Processor receives register read or register write commands from the DCB across the Command I/F.  The commands received are processed with no system timing knowledge (i.e. Super-PPS, PPS, and Bank Switch).

A read command generates a data product which is sent to the DCB via the Data Processor.  One the read command is received, the Command Processor performs a read operation on the Register I/F using the address specified in the command.  The data read is appended with an App ID and forwarded to the Data Processor and queued up to be transmitted to the DCB.

When a write command is received, a write operation is performed on the Register I/F using the address and data specified in the command.

Note: Although the Command Processor is oblivious to any system timing information, write operations made to the Triggers, Waveform, or Spectra/X-Spectra registers take effect only when the PPS is sampled high (i.e. on the PPS event).

### Data Processor

The data processor contains two FIFOs which are used to store the data products received and to transmit the data products that has already been stored in one of the FIFOs to the DCB via the two telemetry signals.  The Data Processor operates such that one FIFO is used to store data products while the other FIFO is used to transmit data products.  The value of the Bank Switch (DT 2), as well as the value of the embedded timing accompanying the Waveform data products (ET 4), determines which FIFO is storing data products vs transmitting data products.  Hence, as the Bank Switch changes polarity every 1/128th of a second the data products are consequently delayed 1/128th of a second by the Data Processor being transmitting to the DCB.

#### Data Only

The data products received on the following interfaces does not contain any timing information:

- TRIG I/F

- SPEC I/F

- DAT I/F

When these data products are received by the Data Processor, they are stored in the FIFO according to the value of the Bank Switch (DT 2).  While these data products are stored in one FIFO, the content of the other FIFO is transmitted out onto the Telemetry I/F.

#### Data and Embedded Timing Info

The WAVE I/F provide the Data Processor with both data products and timing information.  The timing information embedded on the WAVE I/F is the value of Bank Switch inserted into the data stream when the 16KHz sample was read from the ADC.  The value of the embedded Bank Switch is used to select which FIFO will store the Waveform data products rather than external Bank Switch signal (i.e. DT 2).

As the data products generated by the Waveform Player “trickle” in at a user defined data rate, the embedded timing is used in order to compensate for the data path delay through the FPGA.  Hence, ensuring the relative timing between the data product and the PPS generated by the DCM remains constant.

Note that using the embedded timing information will allow the data products to be written into the same FIFO that is being used to transmit data products to the DCB.